======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-101

ASSEMBLY NAME:

Standard TV Outlet

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Standard TV Outlet Assembly establishes the baseline television

infrastructure used throughout Digitally Defined builder homes.

This assembly provides a clean, serviceable, future-ready TV location

that supports standard television connectivity and future upgrade

paths.

======================================================================

ASSEMBLY CATEGORY

Television Infrastructure

======================================================================

STANDARD APPLICATIONS

This assembly may be used in:

• Great Room

• Primary Bedroom

• Standard Bedroom

• Office

• Bonus Room

• Loft

• Guest Suite

• Casita

• Game Room

• Home Gym

• Library

======================================================================

STANDARD SCOPE

This assembly includes:

• One low-voltage TV outlet location

• One 2" ENT conduit path

• One pull string

• One low-voltage mounting bracket

• One finished wall plate

• Cable labeling

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install one 2" ENT conduit from the TV outlet location vertically

through the wall cavity and into the accessible attic space.

Conduit shall terminate above the top plate and remain accessible

for future wiring.

Conduit shall not be routed horizontally across the attic unless

specified by the approved floor plan.

======================================================================

STANDARD HEIGHT

Default TV outlet height:

60" to center

Final height may be adjusted based on:

• Builder standard

• Fireplace location

• Room layout

• Cabinet elevation

• Approved floor plan

======================================================================

REQUIRED MATERIALS

• 2" ENT conduit

• ENT connector

• ENT strap

• Pull string

• Low-voltage mounting bracket

• Wall plate

• Cable label

• Fasteners

• Fire stop where required

======================================================================

REQUIRED LABOR

• Layout location

• Verify height

• Install low-voltage bracket

• Drill top plate

• Install conduit

• Secure conduit

• Install pull string

• Label location

• Photograph installation

• Perform quality check

======================================================================

QUALITY CONTROL

Verify:

□ Correct location

□ Correct height

□ Conduit installed

□ Pull string installed

□ Conduit secured

□ No sharp bends

□ Top plate penetration complete

□ Fire stop completed where required

□ Label installed

□ Photo uploaded

======================================================================

ROOM COMPATIBILITY

Approved Room Standards:

ROOM-001 Great Room

ROOM-002 Primary Bedroom

ROOM-003 Standard Bedroom

ROOM-004 Home Office

ROOM-007 Bonus Room

ROOM-008 Loft

ROOM-019 Home Gym

ROOM-020 Game Room

ROOM-022 Guest Suite

======================================================================

UPGRADE COMPATIBILITY

This assembly supports:

• Cat6 Add-On

• RG6 Add-On

• Dual Cat6 Upgrade

• TV Mount Package

• Concealed Wire Package

• Power Bridge

• Soundbar Package

• Fireplace TV Package

======================================================================

INSTALLATION RULES

1.

Every Standard TV Outlet shall include 2" ENT conduit.

--------------------------------------------------

2.

Every conduit shall include pull string.

--------------------------------------------------

3.

Every conduit shall remain accessible after rough-in.

--------------------------------------------------

4.

Every location shall be labeled.

--------------------------------------------------

5.

Every completed assembly shall be photographed.

--------------------------------------------------

6.

No field changes shall be made without approval.

======================================================================

COMMON ERRORS TO AVOID

• Missing pull string

• Incorrect outlet height

• Conduit not secured

• Sharp conduit bends

• Conduit blocked by framing

• Outlet placed behind studs

• Missing label

• Missing photo documentation

======================================================================

REFERENCED DOCUMENTS

DD-002 Material Library

DD-003 Labor Library

DD-011 Installation Standards

DD-012 Quality Control Standards

DD-058 Assembly Standards Library

ROOM-001 through ROOM-040

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-101

======================================================================