======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-102

ASSEMBLY NAME:

Premium Television Outlet

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Premium Television Outlet Assembly provides a high-end

television installation designed for luxury homes, upgraded

builder packages, and custom installations.

This assembly supports expanded connectivity, concealed wiring,

future technology upgrades, and premium equipment installation.

======================================================================

ASSEMBLY CATEGORY

Television Infrastructure

======================================================================

STANDARD APPLICATIONS

This assembly may be used in:

• Great Room

• Primary Bedroom

• Guest Suite

• Casita

• Game Room

• Media Room

• Home Gym

• Bonus Room

• Loft

• Luxury Living Areas

======================================================================

STANDARD SCOPE

This assembly includes:

• One premium television location

• One 2" ENT conduit

• One recessed media box

• One power bridge or recessed power box

• Two Cat6 home runs

• One RG6 home run

• Pull string

• Cable labeling

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install one 2" ENT conduit from the television outlet to the

accessible attic or structured wiring pathway.

Install one recessed media enclosure centered behind the television.

Install one recessed power bridge directly behind the television in

accordance with electrical code.

Install:

• Two Cat6 home runs

• One RG6 home run

Terminate all cabling at the Structured Wiring Closet unless

otherwise specified.

======================================================================

STANDARD HEIGHT

Default TV Centerline

60"

Adjust only when required by:

• Builder Standard

• Fireplace Design

• Cabinet Elevation

• Customer Approved Layout

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced From:

PROD Library

======================================================================

REQUIRED MATERIALS

• 2" ENT

• ENT Couplings

• ENT Straps

• Pull String

• Recessed Media Box

• Recessed Power Bridge

• Low Voltage Bracket

• Wall Plates

• Cable Labels

• Fire Stop Materials

======================================================================

REQUIRED LABOR

Automatically Generated

LAB Operations Include:

• Layout

• Measurement

• Bracket Installation

• Drill Top Plate

• Install ENT

• Secure ENT

• Pull Cat6

• Pull RG6

• Install Pull String

• Install Media Box

• Label Cables

• Photograph Installation

• QC Inspection

======================================================================

QUALITY CONTROL

Verify:

□ Correct TV Height

□ Recessed Box Installed

□ Power Bridge Installed

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Bend Radius

□ Cable Protection Installed

□ Labels Installed

□ Photo Documentation Complete

======================================================================

ROOM COMPATIBILITY

ROOM-001 Great Room

ROOM-002 Primary Bedroom

ROOM-005 Media Room

ROOM-007 Bonus Room

ROOM-008 Loft

ROOM-019 Home Gym

ROOM-020 Game Room

ROOM-022 Guest Suite

ROOM-023 Casita

ROOM-026 Library

======================================================================

UPGRADE COMPATIBILITY

Compatible With:

• Premium TV Mount

• Articulating Mount

• Soundbar Assembly

• AV Receiver Package

• Dolby Atmos Package

• Fiber Upgrade

• Additional Cat6

• Additional RG6

• Smart TV Package

======================================================================

INSTALLATION RULES

1.

Premium TV locations shall always receive two Cat6

home runs.

--------------------------------------------------

2.

Premium TV locations shall always include one RG6

home run.

--------------------------------------------------

3.

A recessed media enclosure shall always be installed.

--------------------------------------------------

4.

A recessed power solution shall always be installed.

--------------------------------------------------

5.

All cables shall terminate in the Structured Wiring

Closet unless otherwise approved.

--------------------------------------------------

6.

No field modifications shall be made without

engineering approval.

======================================================================

COMMON ERRORS TO AVOID

• Incorrect TV Height

• Missing Recessed Box

• Missing Power Bridge

• ENT Not Continuous

• Cat6 Not Home Run

• RG6 Not Home Run

• Missing Pull String

• Poor Cable Radius

• Missing Labels

• Missing QC Photos

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM-001 through ROOM-040

PROD Library

MAT Library

LAB Library

QC Standards

Installation Standards

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-102

======================================================================