======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-104

ASSEMBLY NAME:

Outdoor Television Outlet

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Television Infrastructure

--------------------------------------------------

Assembly Type

Exterior

--------------------------------------------------

Difficulty Level

4 of 5

--------------------------------------------------

Required Crew

1 Low-Voltage Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

======================================================================

PURPOSE

The Outdoor Television Outlet Assembly establishes the standard

installation for exterior television locations while ensuring

weather resistance, code compliance, long-term reliability, and

future serviceability.

This assembly is designed to support outdoor entertainment systems

installed on covered patios, outdoor kitchens, balconies, and other

approved exterior living spaces.

======================================================================

STANDARD APPLICATIONS

Approved Rooms

• Outdoor Living

• Covered Patio

• Outdoor Kitchen

• Balcony

• Rear Yard Pavilion

• Pool Cabana

• Outdoor Lounge

======================================================================

STANDARD SCOPE

This assembly includes:

• One outdoor television location

• One 2" ENT conduit

• One pull string

• Two Cat6 home runs

• One RG6 home run

• One weatherproof low-voltage enclosure

• Cable labeling

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install one continuous 2" ENT conduit from the television location

to the structured wiring pathway.

Install one weather-rated low-voltage enclosure centered behind the

television mounting location.

Install:

• Two Cat6 home runs

• One RG6 home run

Install all wiring using approved exterior installation practices.

Maintain proper separation from electrical wiring and all heat

sources.

Seal all exterior wall penetrations using approved weatherproof

sealant.

======================================================================

STANDARD LOCATION

Television location shall be centered within the designated outdoor

viewing area unless otherwise shown on approved construction

documents.

Default TV Centerline

60"

Adjust location only when required by:

• Builder Standards

• Fireplace Layout

• Outdoor Kitchen Design

• Customer Approved Drawings

======================================================================

ENVIRONMENTAL REQUIREMENTS

Outdoor television locations shall:

• Be protected from direct water intrusion

• Be located under covered structures whenever possible

• Use UV-resistant materials

• Use weather-rated wall plates and enclosures

• Maintain service accessibility

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

Typical materials include:

• 2" ENT

• Pull String

• Weatherproof LV Box

• UV-Rated Cable Labels

• Exterior Sealant

• Stainless Fasteners

• Fire Stop Materials (where required)

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

Typical operations include:

• Layout

• Height Verification

• Install LV Box

• Install ENT

• Pull Cat6

• Pull RG6

• Install Pull String

• Seal Exterior Penetrations

• Label Cables

• QC Inspection

======================================================================

QUALITY CONTROL

Verify:

□ Correct Location

□ Correct Height

□ ENT Installed

□ Pull String Installed

□ Weatherproof Box Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Exterior Sealant Applied

□ UV-Rated Materials Used

□ Proper Bend Radius

□ Labels Installed

□ Photo Documentation Complete

======================================================================

ROOM COMPATIBILITY

ROOM-006 Outdoor Living

ROOM-024 Outdoor Kitchen

ROOM-032 Rear Yard

ROOM-035 Balcony

Future Outdoor Structures

======================================================================

UPGRADE COMPATIBILITY

Compatible With:

• Premium Outdoor TV Mount

• Outdoor Soundbar

• Outdoor Speaker Package

• Outdoor Subwoofer

• Outdoor Access Point

• Fiber Upgrade

• Additional Cat6

• Smart TV Package

• Weatherproof Equipment Cabinet

======================================================================

INSTALLATION RULES

1.

All exterior components shall be weather-rated for

their intended environment.

--------------------------------------------------

2.

Every Outdoor TV Outlet shall receive two Cat6 home

runs.

--------------------------------------------------

3.

Every Outdoor TV Outlet shall receive one RG6 home

run.

--------------------------------------------------

4.

Every conduit shall include a pull string.

--------------------------------------------------

5.

All exterior penetrations shall be sealed using

approved weatherproof sealants.

--------------------------------------------------

6.

Only approved products and assemblies may be used.

======================================================================

COMMON ERRORS TO AVOID

• Using interior-rated components outdoors

• Missing exterior sealant

• Missing pull string

• Improper conduit support

• Incorrect television height

• Missing cable labels

• Exposed wiring

• Poor weatherproofing

• Improper bend radius

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Installation Standards

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-104

======================================================================