======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-105

ASSEMBLY NAME:

Media Room Television Outlet

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Television Infrastructure

--------------------------------------------------

Assembly Type

Dedicated Home Theater

--------------------------------------------------

Difficulty Level

5 of 5

--------------------------------------------------

Required Crew

1 Low-Voltage Technician

(2 Recommended During Trim)

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-203 Quad Cat6

ASM-305 5.1 Theater

ASM-306 7.1 Theater

ASM-307 Dolby Atmos

======================================================================

PURPOSE

The Media Room Television Outlet Assembly establishes the standard

television infrastructure for dedicated media rooms and home

theaters.

This assembly provides maximum flexibility for large displays,

surround sound systems, gaming consoles, AV receivers, streaming

devices, and future technology upgrades.

======================================================================

STANDARD APPLICATIONS

Approved Rooms

• Media Room

• Home Theater

• Luxury Game Room

• Dedicated Entertainment Room

======================================================================

STANDARD SCOPE

This assembly includes:

• One premium television location

• One 2" ENT conduit

• One recessed media enclosure

• One recessed power bridge

• Four Cat6 home runs

• One RG6 home run

• Pull string

• Cable identification

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install one continuous 2" ENT conduit from the television location

to the structured wiring pathway.

Install one recessed media enclosure centered behind the television.

Install one recessed power bridge in accordance with electrical

code.

Install:

• Four Cat6 home runs

• One RG6 home run

Maintain cable separation from electrical wiring throughout the

installation.

Leave adequate service loops at both ends of all cabling.

======================================================================

STANDARD LOCATION

Television shall be centered on the primary viewing wall.

Default TV Centerline

60"

Adjust only when required by:

• Engineered theater layout

• Customer-approved seating plan

• Project-specific drawings

======================================================================

MEDIA ROOM REQUIREMENTS

Assembly shall support:

• Smart Television

• AV Receiver

• Streaming Device

• Gaming Console

• Future Gaming Console

• Network Switch

• HDMI Distribution

• Control System

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

Typical materials include:

• 2" ENT

• Pull String

• Recessed Media Box

• Recessed Power Bridge

• Low-Voltage Bracket

• Cable Labels

• Fire Stop Materials

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

Typical operations include:

• Layout

• Height Verification

• Install LV Bracket

• Install ENT

• Pull Four Cat6

• Pull RG6

• Install Pull String

• Install Recessed Box

• Label Cables

• Photograph Installation

• QC Inspection

======================================================================

QUALITY CONTROL

Verify:

□ TV centered

□ Correct mounting height

□ ENT installed

□ Pull string installed

□ Recessed media box installed

□ Recessed power bridge installed

□ Four Cat6 installed

□ One RG6 installed

□ Service loops maintained

□ Cable labels installed

□ Photo documentation complete

======================================================================

ROOM COMPATIBILITY

ROOM-005 Media Room

ROOM-020 Game Room

Future Theater Rooms

======================================================================

UPGRADE COMPATIBILITY

Compatible With:

• ASM-305 5.1 Theater

• ASM-306 7.1 Theater

• ASM-307 Dolby Atmos

• Fiber Backbone

• AV Receiver Package

• Equipment Rack

• Premium TV Mount

• Soundbar

• Hidden Equipment Chase

• Control System

======================================================================

INSTALLATION RULES

1.

Media Room television locations shall always receive

four Cat6 home runs.

--------------------------------------------------

2.

Media Room television locations shall always receive

one RG6 home run.

--------------------------------------------------

3.

Every installation shall include one recessed media

enclosure.

--------------------------------------------------

4.

Every installation shall include one recessed power

bridge.

--------------------------------------------------

5.

All wiring shall terminate in the Structured Wiring

Closet unless otherwise approved.

--------------------------------------------------

6.

No field modifications shall be made without

engineering approval.

======================================================================

COMMON ERRORS TO AVOID

• Installing only two Cat6 cables

• Missing recessed media enclosure

• Missing pull string

• No service loops

• Improper cable labeling

• Poor cable routing

• Incorrect TV location

• Poor conduit support

• Missing QC photographs

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Installation Standards

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-105

======================================================================