======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-106

ASSEMBLY NAME:

Projector Location

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Television Infrastructure

--------------------------------------------------

Assembly Type

Dedicated Projection System

--------------------------------------------------

Difficulty Level

5 of 5

--------------------------------------------------

Required Crew

1 Low-Voltage Technician

(2 Recommended During Equipment Installation)

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-203 Quad Cat6

ASM-601 Structured Wiring Panel

======================================================================

PURPOSE

The Projector Location Assembly establishes the standard

infrastructure for ceiling-mounted projection systems installed in

dedicated home theaters, media rooms, training rooms, and premium

entertainment spaces.

The assembly provides sufficient infrastructure for current

projection systems while allowing future equipment replacement

without opening finished walls or ceilings.

======================================================================

STANDARD APPLICATIONS

Approved Rooms

• Media Room

• Home Theater

• Theater Lobby

• Luxury Game Room

• Training Room

• Commercial Presentation Room

======================================================================

STANDARD SCOPE

This assembly includes:

• One ceiling projector location

• One 2" ENT conduit

• One ceiling-mounted low-voltage box

• One projector mounting block location

• Four Cat6 home runs

• One pull string

• Cable identification

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install one 2" ENT conduit from the projector location to the

Structured Wiring Closet.

Install one ceiling-mounted low-voltage enclosure.

Install:

• Four Cat6 home runs

• Pull string

Provide adequate service loop at the projector location.

Projector mounting shall align with the engineered projector throw

distance and screen location.

Blocking for projector mounting shall be installed by the framing

contractor or builder when specified on approved construction

documents.

======================================================================

STANDARD LOCATION

Projector location shall be centered with the projection screen.

Final location shall be determined by:

• Projector manufacturer

• Throw distance

• Screen size

• Ceiling height

• Engineered room layout

No field relocation shall occur without approval.

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

Typical materials include:

• 2" ENT

• Pull String

• Ceiling LV Box

• Cable Labels

• Fire Stop Materials

• Fasteners

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

Typical operations include:

• Layout

• Verify Throw Distance

• Install LV Box

• Install ENT

• Pull Four Cat6

• Install Pull String

• Label Cables

• Photograph Installation

• QC Inspection

======================================================================

REQUIRED TOOLS

• Tape Measure

• Laser Distance Meter

• Drill

• Hole Saw

• Fish Tape

• Conduit Cutter

• Cable Pulling Equipment

• Label Printer

======================================================================

SAFETY & PPE REQUIREMENTS

Required PPE

□ Safety Glasses

□ Cut-Resistant Gloves

□ Hard Hat (Construction Phase)

□ Dust Mask (Attic Access)

□ Ladder Safety Compliance

Safety Requirements

• Verify ceiling framing before drilling.

• Maintain safe ladder positioning.

• Protect existing electrical wiring.

• Follow attic safety procedures.

======================================================================

QUALITY CONTROL

Verify:

□ Projector centered on screen

□ Throw distance verified

□ Ceiling box installed

□ ENT installed

□ Pull string installed

□ Four Cat6 installed

□ Service loop maintained

□ Cable labels installed

□ Fire stop completed

□ Photo documentation complete

======================================================================

INSPECTION & ACCEPTANCE CRITERIA

PASS when:

□ Projector location matches approved plans

□ Conduit is continuous and unobstructed

□ Four Cat6 home runs terminate correctly

□ Pull string operates freely

□ Labels match documentation

□ Photos uploaded to project file

FAIL when:

□ Location differs from engineered layout

□ Missing conduit

□ Missing Cat6

□ Damaged conduit

□ Missing pull string

□ Incomplete documentation

======================================================================

ROOM COMPATIBILITY

ROOM-005 Media Room

ROOM-020 Game Room

ROOM-028 Theater Lobby

Future Theater Designs

======================================================================

UPGRADE COMPATIBILITY

Compatible With:

• Projection Screen Assembly

• Equipment Rack

• Fiber Backbone

• AV Receiver Package

• Dolby Atmos System

• Hidden Equipment Chase

• Home Automation Controller

======================================================================

INSTALLATION RULES

1.

Every projector location shall receive four Cat6

home runs.

--------------------------------------------------

2.

Every projector location shall receive one continuous

2" ENT conduit.

--------------------------------------------------

3.

Projector locations shall follow engineered throw

distance calculations.

--------------------------------------------------

4.

Blocking shall be installed before drywall when

required.

--------------------------------------------------

5.

Service loops shall be maintained at the projector

location.

--------------------------------------------------

6.

No field relocation shall occur without engineering

approval.

======================================================================

COMMON ERRORS TO AVOID

• Incorrect throw distance

• Off-center projector

• Missing blocking

• Missing pull string

• Insufficient service loop

• Missing cable labels

• Poor conduit support

• Missing QC photographs

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Installation Standards

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-106

======================================================================