======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-108

ASSEMBLY NAME:

Recessed Media Box Assembly

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Television Infrastructure

--------------------------------------------------

Assembly Type

Cable Management

--------------------------------------------------

Difficulty Level

3 of 5

--------------------------------------------------

Required Crew

1 Low-Voltage Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-1 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-101

ASM-102

ASM-103

ASM-104

ASM-105

======================================================================

PURPOSE

The Recessed Media Box Assembly provides a concealed,

serviceable enclosure behind wall-mounted televisions

for low-voltage cabling and media device connections.

The assembly allows televisions to mount flush against

the finished wall while protecting cabling and

maintaining accessibility for future service.

======================================================================

STANDARD APPLICATIONS

Approved Rooms

• Great Room

• Primary Bedroom

• Standard Bedroom

• Home Office

• Media Room

• Bonus Room

• Loft

• Guest Suite

• Casita

• Game Room

• Home Gym

• Outdoor Living

(Weather-Rated Version)

======================================================================

STANDARD SCOPE

This assembly includes:

• One recessed media enclosure

• Low-voltage cable management

• Device mounting provisions

• Cable bend radius protection

• Label identification

• Photo documentation

======================================================================

STANDARD INSTALLATION

Install the recessed media enclosure centered behind

the television mounting location.

Verify enclosure depth is compatible with wall

construction.

Provide sufficient cable slack inside the enclosure.

Maintain minimum bend radius for all Cat6, coax,

fiber, and HDMI cables.

Install protective bushings where required.

The enclosure shall remain fully accessible after

television installation.

======================================================================

STANDARD LOCATION

Centered behind television.

Typical mounting elevation:

60" Centerline

Adjust only when approved by:

• Builder Standards

• Fireplace Layout

• Cabinet Design

• Engineered Drawings

======================================================================

SUPPORTED DEVICES

The enclosure shall accommodate:

• HDMI Connections

• Cat6 Connections

• RG6 Connections

• Fiber Connections

• Streaming Devices

• HDMI Baluns

• IR Extenders

• Future Expansion

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

Typical materials include:

• Recessed Media Box

• Mounting Screws

• Bushings

• Cable Labels

• Fasteners

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

Typical operations include:

• Layout

• Verify Stud Location

• Install Media Box

• Verify Depth

• Secure Box

• Label Assembly

• Photograph Installation

• QC Inspection

======================================================================

REQUIRED TOOLS

• Tape Measure

• Level

• Drywall Saw

• Drill

• Screwdrivers

• Label Printer

======================================================================

SAFETY & PPE REQUIREMENTS

Required PPE

□ Safety Glasses

□ Cut-Resistant Gloves

□ Dust Mask

Safety Requirements

• Verify no electrical wiring exists before cutting.

• Protect finished wall surfaces.

• Maintain structural integrity of framing.

======================================================================

QUALITY CONTROL

Verify:

□ Media box centered

□ Correct elevation

□ Flush with finished wall

□ Securely mounted

□ No drywall damage

□ Cable bend radius maintained

□ Cable labels installed

□ Photo documentation complete

======================================================================

INSPECTION & ACCEPTANCE CRITERIA

PASS when:

□ Media box is flush

□ Box is secure

□ Cables accessible

□ Device fits correctly

□ Labels complete

□ Photos uploaded

FAIL when:

□ Crooked installation

□ Loose box

□ Damaged drywall

□ Insufficient cable space

□ Missing labels

======================================================================

ROOM COMPATIBILITY

ROOM-001 Great Room

ROOM-002 Primary Bedroom

ROOM-003 Standard Bedroom

ROOM-004 Home Office

ROOM-005 Media Room

ROOM-006 Outdoor Living

ROOM-007 Bonus Room

ROOM-008 Loft

ROOM-019 Home Gym

ROOM-020 Game Room

ROOM-022 Guest Suite

ROOM-023 Casita

======================================================================

UPGRADE COMPATIBILITY

Compatible With:

• ASM-101 Standard TV Outlet

• ASM-102 Premium TV Outlet

• ASM-103 Fireplace TV

• ASM-104 Outdoor TV

• ASM-105 Media Room TV

• ASM-107 Television Power Bridge

• Hidden Equipment Chase

• Premium TV Mount

• Smart TV Package

======================================================================

INSTALLATION RULES

1.

Every Premium Television Assembly shall utilize an

approved recessed media enclosure.

--------------------------------------------------

2.

The enclosure shall remain fully accessible after

television installation.

--------------------------------------------------

3.

Minimum cable bend radius shall always be maintained.

--------------------------------------------------

4.

Cable labels shall remain visible.

--------------------------------------------------

5.

Only approved products shall be installed.

--------------------------------------------------

6.

No field modifications shall be made without

engineering approval.

======================================================================

COMMON ERRORS TO AVOID

• Crooked installation

• Improper depth

• Damaged drywall

• Crushed cables

• Missing bushings

• Missing labels

• Overfilled enclosure

• Missing QC photographs

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Installation Standards

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ASSEMBLY

ASM-108

======================================================================