======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-305

ASSEMBLY NAME:

In-Wall Speaker Pair Assembly

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Audio Systems

--------------------------------------------------

Assembly Type

Architectural Audio Infrastructure

--------------------------------------------------

Difficulty Level

3 of 5

--------------------------------------------------

Required Crew

1 Level-2 Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-210 Structured Backbone

Builder Framing Standards

======================================================================

PURPOSE

The In-Wall Speaker Pair Assembly establishes the standard

installation for flush-mounted in-wall speakers used in distributed

audio, media rooms, and dedicated listening environments.

The assembly ensures proper structural placement, acoustic

performance, and long-term serviceability while preserving the

finished appearance of the wall.

======================================================================

STANDARD APPLICATIONS

Approved For

• Great Room

• Media Room

• Home Office

• Library

• Game Room

• Flex Room

• Dedicated Listening Room

======================================================================

STANDARD SCOPE

This assembly includes:

• Two in-wall speaker locations

• Two dedicated 14/2 or 16/2 speaker wire home runs

• Mounting locations

• Service loops

• Permanent cable identification

• Documentation

• Photo documentation

======================================================================

AUDIO DESIGN CRITERIA

Configuration

Stereo Pair

--------------------------------------------------

Recommended Mounting Height

Ear Height (Seated)

Unless engineered otherwise.

--------------------------------------------------

Speaker Separation

8'–14'

Depending on room dimensions.

--------------------------------------------------

Wall Clearance

Minimum 16"

From adjacent corners unless engineered otherwise.

--------------------------------------------------

Recommended Amplifier Power

50–150 Watts RMS Per Channel

--------------------------------------------------

Nominal Impedance

8 Ohms

Unless specified by system design.

======================================================================

ASSEMBLY EXECUTION PHASES

PLANNING & ENGINEERING

• Verify furniture layout

• Verify TV location (if applicable)

• Verify framing layout

• Confirm speaker spacing

--------------------------------------------------

MATERIAL VERIFICATION

• Speaker model verified

• Correct speaker wire verified

• Mounting hardware verified

--------------------------------------------------

PREWIRE / ROUGH-IN

• Install dedicated speaker wires

• Protect wire through framing

• Install nail plates where required

• Leave 24" service loops

--------------------------------------------------

TRIM / DEVICE INSTALLATION

• Cut wall openings

• Install speakers

• Verify mounting pressure

• Verify grille alignment

--------------------------------------------------

SYSTEM CONFIGURATION

• Verify amplifier channel assignment

• Verify polarity

• Configure DSP if applicable

--------------------------------------------------

TESTING & COMMISSIONING

• Audio sweep test

• Verify left/right orientation

• Confirm balanced output

• Check for vibration or wall resonance

--------------------------------------------------

QUALITY CONTROL INSPECTION

Verify

□ Speaker spacing correct

□ Mounting secure

□ Wiring labeled

□ Polarity verified

□ Cosmetic finish acceptable

□ Photos complete

--------------------------------------------------

DOCUMENTATION & PROJECT CLOSEOUT

Required

□ Speaker Schedule

□ Amplifier Assignment

□ Room Assignment

□ As-Built Updated

□ Digital Project Record Updated

--------------------------------------------------

WARRANTY & LIFECYCLE

Expected Service Life

20+ Years (Infrastructure)

Replace speakers without disturbing wiring whenever

possible.

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

======================================================================

REQUIRED TOOLS

• Fish Tape

• Laser Measure

• Drywall Saw

• Wire Stripper

• Label Printer

• Stud Finder

• Level

======================================================================

INTER-TRADE COORDINATION

Builder

□ Wall layout approved

--------------------------------------------------

Electrical

□ Outlet conflicts verified

--------------------------------------------------

HVAC

□ Duct conflicts eliminated

--------------------------------------------------

Framing

□ Blocking verified (if required)

--------------------------------------------------

Drywall

□ Cutout locations coordinated

--------------------------------------------------

Painting

□ Finish protection maintained

======================================================================

ROOM COMPATIBILITY

ROOM-001 Great Room

ROOM-004 Home Office

ROOM-005 Media Room

ROOM-020 Game Room

ROOM-026 Library

Future Listening Areas

======================================================================

UPGRADE COMPATIBILITY

Compatible With

ASM-306 Soundbar Prewire

ASM-308 Subwoofer Prewire

ASM-321 Stereo Audio System

ASM-323 3.1 Audio System

ASM-325 5.1 Surround System

ASM-340 Whole-Home Audio Zone

======================================================================

INSTALLATION RULES

1.

Every speaker shall receive a dedicated home run.

--------------------------------------------------

2.

Maintain minimum 24-inch service loop.

--------------------------------------------------

3.

Maintain correct left/right orientation.

--------------------------------------------------

4.

Avoid conflicts with plumbing, electrical, HVAC,

and structural framing.

--------------------------------------------------

5.

Only approved architectural speakers shall be

installed.

--------------------------------------------------

6.

Every completed installation shall include testing,

documentation, and QC photographs.

======================================================================

COMMON ERRORS TO AVOID

• Uneven speaker spacing

• Incorrect mounting height

• Reversed polarity

• Missing labels

• Wall vibration

• Damaged drywall finish

• Missing QC documentation

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Architectural Audio Standards

Builder Standards

Installation Standards

======================================================================

REVISION APPROVAL

Engineering Approval

________________________

Operations Approval

________________________

Effective Date

________________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF ASSEMBLY

ASM-305

======================================================================