======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-306

ASSEMBLY NAME:

Soundbar Prewire Assembly

VERSION:

1.0

STATUS:

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Audio Systems

--------------------------------------------------

Assembly Type

Television Audio Infrastructure

--------------------------------------------------

Difficulty Level

2 of 5

--------------------------------------------------

Required Crew

1 Level-1 Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-1 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-101 Standard TV Outlet

ASM-107 Television Power Bridge

ASM-108 Recessed Media Box

ASM-210 Structured Backbone

======================================================================

PURPOSE

The Soundbar Prewire Assembly establishes the standard

infrastructure required to support active or passive soundbar

installations at wall-mounted television locations.

The assembly provides concealed wiring pathways, proper mounting

coordination, and future flexibility while maintaining a clean,

professional appearance.

======================================================================

STANDARD APPLICATIONS

Approved For

• Great Room

• Media Room

• Primary Bedroom

• Bonus Room

• Game Room

• Home Office

• Outdoor Television (Weather-Rated Design)

======================================================================

STANDARD SCOPE

This assembly includes:

• Soundbar wire pathway

• Audio signal cabling

• Control cabling (if required)

• Conduit for future upgrades

• Service loop

• Cable identification

• Photo documentation

======================================================================

AUDIO DESIGN CRITERIA

Configuration

Single Soundbar

--------------------------------------------------

Typical Mounting Height

4"–8"

Below television

--------------------------------------------------

Horizontal Alignment

Centered with television

--------------------------------------------------

Minimum Clearance

2"

From television mounting hardware

--------------------------------------------------

Future Expansion

Provide pathway for upgraded soundbar or AV receiver.

======================================================================

ASSEMBLY EXECUTION PHASES

PLANNING & ENGINEERING

• Verify television layout

• Verify mounting height

• Verify recessed media box location

• Confirm conduit pathway

--------------------------------------------------

MATERIAL VERIFICATION

• Speaker cable verified

• HDMI cable verified (if required)

• Conduit verified

• Labels verified

--------------------------------------------------

PREWIRE / ROUGH-IN

• Install speaker wiring

• Install conduit

• Install pull string

• Leave 24" service loop

• Protect cable through framing

--------------------------------------------------

TRIM / DEVICE INSTALLATION

• Verify soundbar mounting location

• Route cables through recessed media box

• Secure cable management

--------------------------------------------------

SYSTEM CONFIGURATION

• Assign amplifier output (if passive)

• Verify ARC/eARC routing (if active)

• Configure control interface (if applicable)

--------------------------------------------------

TESTING & COMMISSIONING

• Verify audio signal

• Verify left/right orientation

• Verify cable continuity

• Confirm serviceability

--------------------------------------------------

QUALITY CONTROL INSPECTION

Verify

□ Wiring complete

□ Service loop present

□ Labels installed

□ Proper routing maintained

□ Photos complete

--------------------------------------------------

DOCUMENTATION & PROJECT CLOSEOUT

Required

□ Wiring Schedule

□ Equipment Assignment

□ TV Location Reference

□ As-Built Updated

□ Digital Project Record Updated

--------------------------------------------------

WARRANTY & LIFECYCLE

Expected Service Life

20+ Years (Infrastructure)

Replace soundbar without disturbing wiring.

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

======================================================================

REQUIRED TOOLS

• Fish Tape

• Wire Stripper

• Label Printer

• Drill

• Laser Measure

======================================================================

INTER-TRADE COORDINATION

Builder

□ TV wall approved

--------------------------------------------------

Electrical

□ Power bridge coordinated

--------------------------------------------------

Framing

□ TV blocking verified

--------------------------------------------------

Drywall

□ Media box opening verified

--------------------------------------------------

Finish Carpentry

□ Mantel or cabinetry clearances verified

======================================================================

ROOM COMPATIBILITY

ROOM-001 Great Room

ROOM-002 Primary Bedroom

ROOM-005 Media Room

ROOM-007 Bonus Room

ROOM-020 Game Room

ROOM-023 Casita

======================================================================

UPGRADE COMPATIBILITY

Compatible With

ASM-307 Passive Soundbar

ASM-308 Subwoofer Prewire

ASM-312 Audio Equipment Cabinet

ASM-321 Stereo Audio System

ASM-323 3.1 Audio System

ASM-325 5.1 Surround System

======================================================================

INSTALLATION RULES

1.

Every soundbar location shall include a concealed

wire pathway.

--------------------------------------------------

2.

Maintain minimum 24-inch service loop.

--------------------------------------------------

3.

Soundbar shall remain centered beneath the television.

--------------------------------------------------

4.

Provide conduit for future upgrades whenever possible.

--------------------------------------------------

5.

Only approved low-voltage cabling shall be installed.

--------------------------------------------------

6.

Every completed installation shall include testing,

documentation, and QC photographs.

======================================================================

COMMON ERRORS TO AVOID

• Missing conduit

• No service loop

• Incorrect soundbar location

• Cable conflicts with TV mount

• Missing labels

• Poor cable routing

• Missing QC documentation

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Television Standards

Audio Standards

Installation Standards

======================================================================

REVISION APPROVAL

Engineering Approval

________________________

Operations Approval

________________________

Effective Date

________________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF ASSEMBLY

ASM-306

======================================================================