======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID:

ASM-308

ASSEMBLY CLASSIFICATION CODE

ACC-AUD-SUB-PRE-001

--------------------------------------------------

ASSEMBLY NAME

Subwoofer Prewire Assembly

VERSION

1.0

STATUS

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Audio Systems

--------------------------------------------------

Assembly Type

Low-Frequency Audio Infrastructure

--------------------------------------------------

Difficulty Level

2 of 5

--------------------------------------------------

Required Crew

1 Level-1 Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-1 Technician

--------------------------------------------------

Dependencies

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

ASM-210 Structured Backbone

ASM-306 Soundbar Prewire

======================================================================

PURPOSE

The Subwoofer Prewire Assembly establishes the standard

infrastructure for powered subwoofer locations in home theater,

media room, and television audio systems.

The assembly provides concealed signal pathways, power coordination,

and placement flexibility to achieve consistent low-frequency

performance while allowing future equipment upgrades.

======================================================================

STANDARD APPLICATIONS

Approved For

• Great Room

• Media Room

• Home Theater

• Bonus Room

• Game Room

• Dedicated Listening Room

======================================================================

STANDARD SCOPE

This assembly includes:

• One shielded subwoofer signal cable (or approved equivalent)

• One Category cable for future control (optional per design)

• Conduit for future upgrades

• Pull string

• Service loops

• Cable identification

• Photo documentation

======================================================================

AUDIO DESIGN CRITERIA

Configuration

Single Powered Subwoofer

--------------------------------------------------

Recommended Placement

Front wall corner

OR

Front one-third wall position

Final placement determined during commissioning.

--------------------------------------------------

Recommended Frequency Range

20 Hz – 120 Hz

--------------------------------------------------

Future Expansion

Provide conduit sized for future cable replacement.

======================================================================

ASSEMBLY EXECUTION PHASES

PLANNING & ENGINEERING

• Verify room layout

• Verify furniture layout

• Verify television location

• Select preliminary subwoofer location

--------------------------------------------------

MATERIAL VERIFICATION

• Shielded signal cable verified

• Conduit verified

• Pull string verified

• Labels verified

--------------------------------------------------

PREWIRE / ROUGH-IN

• Install conduit

• Install pull string

• Pull shielded signal cable

• Install optional Category cable if specified

• Leave 24" service loop

--------------------------------------------------

TRIM / DEVICE INSTALLATION

• Install wall plate

• Verify cable accessibility

• Protect unused conduit

--------------------------------------------------

SYSTEM CONFIGURATION

• Assign subwoofer output

• Configure AVR or DSP

• Verify crossover settings

--------------------------------------------------

TESTING & COMMISSIONING

• Verify signal continuity

• Verify proper output

• Confirm polarity

• Perform initial placement evaluation

--------------------------------------------------

PERFORMANCE VERIFICATION

□ Signal path verified

□ No induced noise or hum

□ Proper crossover operation

□ Proper bass integration

□ No audible distortion

□ Meets design intent

--------------------------------------------------

QUALITY CONTROL INSPECTION

Verify

□ Correct location

□ Conduit installed

□ Pull string installed

□ Service loop installed

□ Labels complete

□ Photos complete

--------------------------------------------------

DOCUMENTATION & PROJECT CLOSEOUT

Required

□ Cable Schedule

□ Equipment Assignment

□ Room Assignment

□ As-Built Updated

□ Digital Project Record Updated

--------------------------------------------------

WARRANTY & LIFECYCLE

Expected Service Life

25+ Years (Infrastructure)

Signal cabling shall remain serviceable independent

of the connected equipment.

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from:

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from:

MAT Library

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from:

LAB Library

======================================================================

REQUIRED TOOLS

• Fish Tape

• Wire Pulling Equipment

• Label Printer

• Cable Tester

• Laser Measure

======================================================================

INTER-TRADE COORDINATION

Builder

□ Furniture layout approved

--------------------------------------------------

Electrical

□ Dedicated receptacle verified

--------------------------------------------------

HVAC

□ Register conflicts eliminated

--------------------------------------------------

Cabinetry

□ Built-in conflicts verified

--------------------------------------------------

Drywall

□ Wall plate location verified

======================================================================

ROOM COMPATIBILITY

ROOM-001 Great Room

ROOM-005 Media Room

ROOM-007 Bonus Room

ROOM-020 Game Room

Future Home Theater Rooms

======================================================================

UPGRADE COMPATIBILITY

Compatible With

ASM-307 Passive Soundbar

ASM-309 Dual Subwoofer Prewire

ASM-323 3.1 Audio System

ASM-325 5.1 Surround System

ASM-327 Dolby Atmos System

ASM-360 Audio Calibration & Commissioning

======================================================================

INSTALLATION RULES

1.

Every subwoofer location shall include a concealed

signal pathway.

--------------------------------------------------

2.

Every conduit shall contain a pull string.

--------------------------------------------------

3.

Provide a minimum 24-inch service loop.

--------------------------------------------------

4.

Coordinate with electrical for dedicated power.

--------------------------------------------------

5.

Only approved signal cabling shall be installed.

--------------------------------------------------

6.

Every completed installation shall include QC

photographs, documentation, and performance

verification.

======================================================================

COMMON ERRORS TO AVOID

• Missing conduit

• Missing pull string

• No nearby power receptacle

• Poor preliminary placement

• Excessively long exposed cables

• Missing labels

• Missing QC documentation

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

Audio Engineering Standards

Installation Standards

======================================================================

REVISION APPROVAL

Engineering Approval

________________________

Operations Approval

________________________

Effective Date

________________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF ASSEMBLY

ASM-308

======================================================================