======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID

ASM-407

--------------------------------------------------

ASSEMBLY CLASSIFICATION CODE

ACC-VID-AVIP-001

--------------------------------------------------

ASSEMBLY NAME

Video over IP Assembly

VERSION

1.0

STATUS

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Video Systems

--------------------------------------------------

Assembly Type

AV over IP Distribution System

--------------------------------------------------

Difficulty Level

5 of 5

--------------------------------------------------

Required Crew

1 Level-2+ Technician

1 Network Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2+

--------------------------------------------------

Dependencies

ASM-210 Structured Backbone

ASM-312 Audio Equipment Cabinet Ecosystem

ASM-404 Video Distribution Assembly

ASM-405 Video Matrix Switching Assembly

ASM-406 HDMI Extension Assembly

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

======================================================================

PURPOSE

The Video over IP Assembly establishes the engineering

standard for distributing high-definition and ultra-high-

definition video across managed Ethernet networks using

AV-over-IP technology.

The assembly standardizes encoder and decoder deployment,

network architecture, bandwidth management, signal integrity,

automation integration, documentation, commissioning, and

future scalability.

======================================================================

ENGINEERING ASSUMPTIONS

This assembly assumes:

• Managed Gigabit or Multi-Gig Ethernet network installed.

• Enterprise-grade managed switches deployed.

• Structured Category cabling certified.

• Dedicated AV equipment rack installed.

• Approved Digitally Defined products utilized.

• Installation performed by qualified technicians.

======================================================================

STANDARD APPLICATIONS

Approved For

• Luxury Residential

• Estate Homes

• Commercial Offices

• Conference Centers

• Hospitality Facilities

• Digital Signage

• Education Facilities

======================================================================

STANDARD SCOPE

Includes

• AV-over-IP Encoders

• AV-over-IP Decoders

• Managed Network Switches

• Network Configuration

• Video Routing

• Automation Integration

• Documentation

• Commissioning

======================================================================

DISPLAY PERFORMANCE REQUIREMENTS (DPR)

Supported Resolutions

HD

Full HD

4K UHD

8K Ready (where supported)

--------------------------------------------------

HDR Formats

HDR10

HDR10+

HLG

Dolby Vision (where supported)

--------------------------------------------------

Frame Rate

Per engineering design

--------------------------------------------------

Compression

Per approved system design

--------------------------------------------------

Image Quality

Equivalent to approved system specifications

======================================================================

SYSTEM DESIGN PERFORMANCE REQUIREMENTS (SDPR)

Availability

24/7 Continuous Operation

--------------------------------------------------

Reliability

Enterprise-grade continuous service

--------------------------------------------------

Latency

Within manufacturer specifications

--------------------------------------------------

Expandability

Additional encoders and decoders supported

--------------------------------------------------

Maintenance Accessibility

Rack-front serviceable

--------------------------------------------------

Acceptance Criteria

Every encoder shall successfully communicate with

every assigned decoder while maintaining approved

video quality and synchronization.

======================================================================

VIDEO NETWORK PERFORMANCE

Encoder Communication

Verified

--------------------------------------------------

Decoder Communication

Verified

--------------------------------------------------

Multicast Routing

Verified

--------------------------------------------------

Bandwidth Utilization

Within engineering design limits

--------------------------------------------------

Packet Loss

Zero during normal operation

--------------------------------------------------

Synchronization

Verified across all assigned displays

======================================================================

CONTROL & AUTOMATION INTEGRATION

Supported Platforms

• Control4

• Savant

• Josh.ai

• Crestron Home

• RTI

• URC

--------------------------------------------------

Supported Interfaces

• IP

• REST API

• Manufacturer Drivers

--------------------------------------------------

Automation Functions

• Video Routing

• Source Selection

• Display Control

• Scene Activation

• Scheduling

======================================================================

SYSTEM CAPACITY & GROWTH PLANNING (SCGP)

Network Capacity

Minimum 30% available bandwidth

--------------------------------------------------

Switch Capacity

Minimum 25% available ports

--------------------------------------------------

Rack Capacity

Minimum 30% reserve

--------------------------------------------------

Infrastructure

Spare conduit recommended

--------------------------------------------------

Future Expansion

Additional encoders

Additional decoders

Additional displays

Additional network switches

======================================================================

CYBERSECURITY & REMOTE ACCESS REQUIREMENTS (CSRR)

Administrative Access

Restricted to authorized personnel

--------------------------------------------------

Authentication

Unique administrator credentials required

--------------------------------------------------

Firmware

Current manufacturer-supported versions

--------------------------------------------------

Remote Access

Encrypted remote access only

--------------------------------------------------

Configuration Backup

Required before firmware updates

--------------------------------------------------

Network Security

Dedicated AV VLAN required

QoS implemented

IGMP Snooping enabled where required

======================================================================

ASSEMBLY EXECUTION PHASES

PLANNING & ENGINEERING

• Verify network topology

• Verify switch capacity

• Verify bandwidth calculations

• Verify endpoint schedule

--------------------------------------------------

MATERIAL VERIFICATION

• Encoders verified

• Decoders verified

• Network switches verified

• Cabling verified

--------------------------------------------------

PREWIRE / ROUGH-IN

• Verify Category cable certification

• Verify conduit

• Verify rack pathways

• Verify permanent labeling

--------------------------------------------------

TRIM / DEVICE INSTALLATION

• Install encoders

• Install decoders

• Connect network

• Connect displays

--------------------------------------------------

SYSTEM CONFIGURATION

• Configure VLANs

• Configure multicast

• Configure QoS

• Configure routing

• Configure automation

• Update firmware

--------------------------------------------------

TESTING & COMMISSIONING

• Verify every encoder

• Verify every decoder

• Verify multicast routing

• Verify bandwidth

• Verify automation

• Verify display synchronization

--------------------------------------------------

PERFORMANCE VERIFICATION

□ Encoders operational

□ Decoders operational

□ Video routing verified

□ Network performance verified

□ Multicast verified

□ Automation verified

□ Meets engineering design intent

--------------------------------------------------

QUALITY CONTROL INSPECTION

Verify

□ Rack organized

□ Cabling labeled

□ Firmware current

□ Configuration backups completed

□ Documentation complete

□ Photos complete

--------------------------------------------------

DOCUMENTATION & PROJECT CLOSEOUT

Required

□ Encoder Schedule

□ Decoder Schedule

□ IP Address Schedule

□ VLAN Schedule

□ Switch Configuration

□ Bandwidth Report

□ Firmware Inventory

□ Configuration Backup

□ As-Built Updated

□ Digital Project Record Updated

--------------------------------------------------

WARRANTY & LIFECYCLE

Infrastructure

25+ Years

Electronics

Manufacturer Warranty

Software

Maintain supported firmware

Annual network health verification recommended.

======================================================================

SIGNAL CLASSIFICATION

Primary Signals

• AV over IP

• Ethernet

• Fiber Ethernet

• HDMI

• USB (where applicable)

• Control

• AC Power

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from

MAT Library

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from

LAB Library

======================================================================

REQUIRED TOOLS

• Network Analyzer

• Cable Certification Tester

• HDMI Analyzer

• Managed Switch Configuration Software

• Laptop

• Label Printer

======================================================================

INTER-TRADE COORDINATION

Builder

□ Equipment rooms approved

--------------------------------------------------

Electrical

□ Dedicated circuits verified

--------------------------------------------------

Network

□ Managed switches configured

□ VLANs verified

□ QoS verified

--------------------------------------------------

Automation

□ Driver compatibility verified

======================================================================

ROOM COMPATIBILITY

Compatible with all applicable ROOM standards.

======================================================================

UPGRADE COMPATIBILITY

ASM-408 Outdoor Television Assembly

ASM-409 Video Calibration & Commissioning

ASM-701 Smart Home Integration

Commercial AV Expansion Standards

======================================================================

INSTALLATION RULES

1.

Dedicated AV VLANs shall be used for all AV-over-IP

devices unless otherwise approved by engineering.

--------------------------------------------------

2.

All network endpoints shall receive permanent labels

and documented IP addresses.

--------------------------------------------------

3.

Managed switches shall be configured according to

approved network standards.

--------------------------------------------------

4.

Firmware shall be updated prior to commissioning.

--------------------------------------------------

5.

Only approved Digitally Defined products shall be

installed.

--------------------------------------------------

6.

Every installation shall include network

configuration backups, commissioning reports,

QC photographs, and updated documentation.

======================================================================

COMMON ERRORS TO AVOID

• Improper VLAN configuration

• Missing QoS configuration

• Incorrect multicast settings

• Duplicate IP addresses

• Inadequate network bandwidth

• Missing firmware updates

• Incomplete documentation

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

AV-over-IP Standards

Ethernet Standards

Network Engineering Standards

Automation Standards

Installation Standards

======================================================================

REVISION APPROVAL

Engineering Approval

______________________

Operations Approval

______________________

Effective Date

______________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF ASSEMBLY

ASM-407

======================================================================