======================================================================

DIGITALLY DEFINED

ASSEMBLY STANDARDS LIBRARY

ASSEMBLY ID

ASM-506

--------------------------------------------------

ASSEMBLY CLASSIFICATION CODE

ACC-NET-PP-001

--------------------------------------------------

ASSEMBLY NAME

Patch Panel Assembly

VERSION

1.0

STATUS

FROZEN

======================================================================

ASSEMBLY METADATA

Assembly Category

Network Systems

--------------------------------------------------

Assembly Type

Structured Cabling Termination

--------------------------------------------------

Difficulty Level

4 of 5

--------------------------------------------------

Required Crew

1 Level-2 Technician

1 Level-1 Technician

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2

--------------------------------------------------

Dependencies

ASM-210 Structured Backbone

ASM-501 Network Rack Assembly

ASM-503 Managed Network Switch Assembly

PROD Library

MAT Library

LAB Library

ROOM Library

QC Library

======================================================================

PURPOSE

The Patch Panel Assembly establishes the engineering standard

for terminating, organizing, documenting, certifying, and

maintaining structured copper and fiber cabling within the

Digitally Defined network infrastructure.

The assembly standardizes termination quality, rack

organization, labeling conventions, certification testing,

serviceability, and future expansion while providing the

foundation for all network-connected systems.

======================================================================

ENGINEERING ASSUMPTIONS

This assembly assumes:

• Structured cabling installed.

• Equipment rack installed.

• Cable routing complete.

• Approved Digitally Defined products utilized.

• Installation performed by qualified technicians.

======================================================================

STANDARD APPLICATIONS

Approved For

• Residential Equipment Rooms

• Commercial MDF

• Commercial IDF

• Telecommunications Rooms

• AV Equipment Rooms

• Security Equipment Rooms

======================================================================

STANDARD SCOPE

Includes

• Copper Patch Panels

• Fiber Patch Panels

• Keystone Modules

• Cable Management

• Labeling

• Certification Testing

• Documentation

• Commissioning

======================================================================

NETWORK PERFORMANCE REQUIREMENTS (NPR)

Supported Cabling

Category 6

Category 6A

Category 8 (where specified)

OM3 Fiber

OM4 Fiber

Single-Mode Fiber

--------------------------------------------------

Bandwidth

Per installed cable category

--------------------------------------------------

Termination Quality

Certified

--------------------------------------------------

Insertion Loss

Within TIA standards

--------------------------------------------------

Return Loss

Within TIA standards

--------------------------------------------------

Availability

24/7 Passive Infrastructure

======================================================================

SYSTEM DESIGN PERFORMANCE REQUIREMENTS (SDPR)

Availability

Continuous Infrastructure

--------------------------------------------------

Reliability

Permanent structured cabling system

--------------------------------------------------

Expandability

Minimum 20% spare ports

--------------------------------------------------

Maintenance Accessibility

Front-service rack access

--------------------------------------------------

Acceptance Criteria

Every cable shall terminate correctly, be

permanently labeled, and pass certification testing.

======================================================================

TERMINATION STANDARDS

Copper Standard

TIA-568.2-D

--------------------------------------------------

Fiber Standard

TIA-568.3-D

--------------------------------------------------

Termination Method

Approved manufacturer system

--------------------------------------------------

Pair Integrity

Verified

--------------------------------------------------

Shield Continuity

Verified where applicable

--------------------------------------------------

Color Code

T568B unless engineering specifies otherwise

======================================================================

LABELING REQUIREMENTS

Patch Panel ID

Required

--------------------------------------------------

Port Number

Required

--------------------------------------------------

Destination Identifier

Required

--------------------------------------------------

Cable Identifier

Required

--------------------------------------------------

Machine Printed Labels Only

Required

======================================================================

SYSTEM CAPACITY & GROWTH PLANNING (SCGP)

Available Ports

Minimum 20% spare

--------------------------------------------------

Rack Capacity

Minimum 30% reserve

--------------------------------------------------

Cable Slack

Maintained per engineering standards

--------------------------------------------------

Future Expansion

Additional patch panels

Additional switches

Fiber expansion

AV expansion

======================================================================

CYBERSECURITY & REMOTE ACCESS REQUIREMENTS (CSRR)

Physical Access

Restricted to authorized personnel

--------------------------------------------------

Rack Security

Maintained

--------------------------------------------------

Documentation

Cable records maintained

--------------------------------------------------

Asset Tracking

Patch panel inventory maintained

======================================================================

ASSEMBLY EXECUTION PHASES

PLANNING & ENGINEERING

• Verify cable schedule

• Verify rack elevation

• Verify labeling plan

• Verify spare capacity

--------------------------------------------------

MATERIAL VERIFICATION

• Patch panels verified

• Keystone jacks verified

• Fiber adapters verified

• Cable managers verified

--------------------------------------------------

PREWIRE / ROUGH-IN

• Verify cable routing

• Verify bend radius

• Verify service loops

• Verify cable identification

--------------------------------------------------

TRIM / DEVICE INSTALLATION

• Install patch panels

• Terminate copper cables

• Terminate fiber cables

• Install cable management

--------------------------------------------------

SYSTEM CONFIGURATION

• Verify labeling

• Verify documentation

• Update rack elevation

--------------------------------------------------

TESTING & COMMISSIONING

• Wiremap verification

• Length verification

• NEXT verification

• Return loss verification

• Fiber loss verification

• Certification reports generated

--------------------------------------------------

PERFORMANCE VERIFICATION

□ All terminations certified

□ Labels complete

□ Wiremap verified

□ Fiber verified

□ Rack organization complete

□ Documentation complete

□ Meets engineering design intent

--------------------------------------------------

QUALITY CONTROL INSPECTION

Verify

□ Bend radius maintained

□ Cable management complete

□ Labels complete

□ Certification reports complete

□ Documentation complete

□ Photos complete

--------------------------------------------------

DOCUMENTATION & PROJECT CLOSEOUT

Required

□ Cable Schedule

□ Patch Panel Schedule

□ Port Assignment Schedule

□ Certification Reports

□ Rack Elevation

□ Fiber Test Reports

□ As-Built Updated

□ Digital Project Record Updated

--------------------------------------------------

WARRANTY & LIFECYCLE

Infrastructure

25+ Years

Terminations

Manufacturer Warranty

Certification

Retained for project lifecycle

======================================================================

SIGNAL CLASSIFICATION

Primary Signals

• Ethernet

• PoE

• Fiber Ethernet

• Telephone (where applicable)

• Control

======================================================================

REQUIRED PRODUCTS

Automatically Generated

Referenced from

PROD Library

======================================================================

REQUIRED MATERIALS

Automatically Generated

Referenced from

MAT Library

======================================================================

REQUIRED LABOR

Automatically Generated

Referenced from

LAB Library

======================================================================

REQUIRED TOOLS

• Cable Certification Tester

• Fiber OTDR (where applicable)

• Fiber Inspection Scope

• Punch Down Tool

• Termination Tools

• Label Printer

======================================================================

INTER-TRADE COORDINATION

Network

□ Switch assignments verified

--------------------------------------------------

Builder

□ Equipment room complete

--------------------------------------------------

Electrical

□ Rack grounding verified

--------------------------------------------------

Security

□ Device assignments verified

======================================================================

ROOM COMPATIBILITY

ROOM-009 Equipment Room

ROOM-010 Network Closet

Commercial MDF

Commercial IDF

Compatible with all applicable ROOM standards.

======================================================================

UPGRADE COMPATIBILITY

ASM-507 UPS & Power Conditioning Assembly

ASM-508 Network Monitoring & Remote Management Assembly

ASM-509 Network Commissioning & Certification Assembly

Future Fiber Infrastructure Standards

======================================================================

INSTALLATION RULES

1.

Every permanent cable shall terminate on an approved

patch panel unless otherwise approved by engineering.

--------------------------------------------------

2.

Machine-printed labels shall be installed on every

patch panel port and cable.

--------------------------------------------------

3.

Copper terminations shall utilize the T568B standard

unless otherwise specified.

--------------------------------------------------

4.

Every cable shall pass certification before project

turnover.

--------------------------------------------------

5.

Only approved Digitally Defined products shall be

installed.

--------------------------------------------------

6.

Every installation shall include certification

reports, QC photographs, rack elevations, and

updated documentation.

======================================================================

COMMON ERRORS TO AVOID

• Mixed termination standards

• Handwritten labels

• Excessive cable untwist

• Improper bend radius

• Missing certification reports

• Poor cable management

• Incomplete documentation

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

ROOM Library

PROD Library

MAT Library

LAB Library

QC Library

TIA-568 Standards

BICSI Best Practices

Structured Cabling Standards

Fiber Optic Standards

Installation Standards

======================================================================

REVISION APPROVAL

Engineering Approval

______________________

Operations Approval

______________________

Effective Date

______________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF ASSEMBLY

ASM-506

======================================================================