======================================================================

DIGITALLY DEFINED

SERVICE STANDARDS LIBRARY

STANDARD ID

DD-0601

--------------------------------------------------

STANDARD CLASSIFICATION CODE

SER-PM-001

--------------------------------------------------

STANDARD NAME

Preventative Maintenance Inspection Standard

VERSION

1.0

STATUS

FROZEN

======================================================================

STANDARD METADATA

Standard Category

Service Standards

--------------------------------------------------

Standard Type

Preventative Maintenance

--------------------------------------------------

Difficulty Level

3 of 5

--------------------------------------------------

Required Crew

1 Level-2 Technician

(2 Technicians for Enterprise Systems)

--------------------------------------------------

Estimated Labor

Automatically Generated

Referenced from LAB Library

--------------------------------------------------

Required Skill Level

Level-2

--------------------------------------------------

Dependencies

DD-058 Assembly Standards Library

QC Library

ROOM Library

LAB Library

Customer Service Agreements

Asset Register

Digital Project Record

======================================================================

PURPOSE

The Preventative Maintenance Inspection Standard

establishes the Digitally Defined engineering and

service requirements for performing scheduled

preventative maintenance inspections on installed

low-voltage systems.

The objective is to maximize system reliability,

identify developing issues before failure,

maintain manufacturer warranty requirements,

extend equipment life, and provide customers with

documented system health assessments.

======================================================================

SERVICE OBJECTIVES

Prevent equipment failures.

--------------------------------------------------

Maintain peak system performance.

--------------------------------------------------

Identify deficiencies before they become service

calls.

--------------------------------------------------

Maintain accurate asset documentation.

--------------------------------------------------

Protect customer investment.

--------------------------------------------------

Support long-term recurring service agreements.

======================================================================

STANDARD APPLICATIONS

Approved For

• Audio Systems

• Video Systems

• Surveillance Systems

• Access Control

• Intrusion Systems

• Networking Equipment

• Building Automation

• Equipment Racks

• UPS Systems

• Fiber Infrastructure

======================================================================

SERVICE FREQUENCY

Standard

Annual

--------------------------------------------------

High Usage Facilities

Semi-Annual

--------------------------------------------------

Mission Critical Facilities

Quarterly

--------------------------------------------------

Customer Specific

Per Service Agreement

======================================================================

PRE-SERVICE REQUIREMENTS

Before Dispatch

□ Review customer history

□ Review previous PM reports

□ Review outstanding deficiencies

□ Verify customer contact

□ Verify scheduled appointment

□ Verify required parts

□ Verify required firmware

□ Verify required test equipment

======================================================================

REQUIRED TOOLS

• Laptop

• Label Printer

• Digital Multimeter

• Network Tester

• Fiber Inspection Kit (if applicable)

• Camera Test Monitor

• Cleaning Supplies

• Firmware Repository

• Digital Inspection Forms

======================================================================

SITE ARRIVAL PROCEDURE

1.

Check in with customer representative.

--------------------------------------------------

2.

Review maintenance scope.

--------------------------------------------------

3.

Confirm operational hours.

--------------------------------------------------

4.

Verify work authorization.

--------------------------------------------------

5.

Identify restricted areas.

--------------------------------------------------

6.

Review any known issues with customer.

======================================================================

PREVENTATIVE MAINTENANCE INSPECTION

General Inspection

□ Equipment clean

□ Equipment securely mounted

□ Labels legible

□ Covers installed

□ No physical damage

□ No water intrusion

□ No corrosion

□ No loose hardware

--------------------------------------------------

Power Inspection

□ UPS operational

□ Power supplies operational

□ Battery health verified

□ Surge protection verified

□ Grounding verified

--------------------------------------------------

Cable Inspection

□ No damaged cable

□ No excessive bend radius

□ Cable support intact

□ Patch cords organized

□ Labels verified

--------------------------------------------------

Network Inspection

□ Devices online

□ Firmware current

□ Ports operational

□ Error logs reviewed

□ Backups verified

--------------------------------------------------

System Operation

□ System operational

□ Alarm conditions reviewed

□ Event logs reviewed

□ Performance verified

□ User accounts verified

--------------------------------------------------

Environmental Inspection

□ Temperature acceptable

□ Ventilation unobstructed

□ Dust accumulation removed

□ Rack airflow acceptable

□ Humidity acceptable

======================================================================

SYSTEM SPECIFIC INSPECTIONS

Surveillance

□ Camera focus

□ Image quality

□ Recording verified

□ Time synchronization

□ IR operation

--------------------------------------------------

Access Control

□ Card readers tested

□ Door contacts tested

□ Lock operation verified

□ REX tested

□ Credentials reviewed

--------------------------------------------------

Intrusion

□ Sensors tested

□ Sirens verified

□ Communications verified

□ Battery health verified

--------------------------------------------------

Audio / Video

□ Signal quality verified

□ DSP operational

□ Displays tested

□ Speaker operation verified

--------------------------------------------------

Networking

□ Link status verified

□ Switch health verified

□ Firewall health reviewed

□ UPS communication verified

======================================================================

FIRMWARE & SOFTWARE REVIEW

Verify

□ Firmware current

□ Security patches current

□ Software licensing current

□ Backup completed

--------------------------------------------------

Updates

Only perform with customer approval unless

covered by service agreement.

======================================================================

CUSTOMER COMMUNICATION

Discuss

• Findings

• Recommendations

• Deficiencies

• Future concerns

• Equipment nearing end-of-life

• Recommended upgrades

======================================================================

DEFICIENCY CLASSIFICATION

Priority 1

Critical

Immediate repair recommended.

--------------------------------------------------

Priority 2

Major

Repair within 30 days.

--------------------------------------------------

Priority 3

Minor

Monitor or repair during next scheduled visit.

--------------------------------------------------

Priority 4

Advisory

Improvement recommendation only.

======================================================================

QUALITY CONTROL

Verify

□ Inspection completed

□ Required testing completed

□ Photos captured

□ Firmware documented

□ Asset inventory verified

□ Customer informed

======================================================================

DOCUMENTATION

Required

□ Preventative Maintenance Report

□ Deficiency Report

□ Firmware Inventory

□ Asset Changes

□ QC Photographs

□ Service Recommendations

□ Customer Signature

□ Digital Project Record Updated

======================================================================

CUSTOMER SIGNOFF

Review

□ Work performed

□ Findings reviewed

□ Recommendations explained

□ Questions answered

□ Customer signature obtained

======================================================================

SERVICE COMPLETION REQUIREMENTS

Before Leaving

□ System fully operational

□ Work area clean

□ Customer satisfied

□ Documentation uploaded

□ Ticket closed

======================================================================

KEY PERFORMANCE INDICATORS (KPIs)

Inspection Completion Rate

--------------------------------------------------

Repeat Failure Rate

--------------------------------------------------

Customer Satisfaction Score

--------------------------------------------------

First-Time Completion Rate

--------------------------------------------------

PM Compliance Rate

--------------------------------------------------

Documentation Accuracy

======================================================================

INSTALLATION / SERVICE RULES

1.

Every preventative maintenance visit shall follow

this inspection standard.

--------------------------------------------------

2.

Critical deficiencies shall be communicated to the

customer before leaving the site.

--------------------------------------------------

3.

No firmware updates shall be performed without

approved authorization unless covered under a

service agreement.

--------------------------------------------------

4.

All maintenance activity shall be documented in the

Digital Project Record.

--------------------------------------------------

5.

Only approved Digitally Defined procedures shall be

used.

--------------------------------------------------

6.

Every preventative maintenance visit shall conclude

with a completed inspection report, customer

review, and updated asset documentation.

======================================================================

COMMON ERRORS TO AVOID

• Skipping inspection steps

• Missing photographs

• Outdated firmware documentation

• Failure to notify customer of deficiencies

• Incomplete inspection reports

• Poor cable management after servicing

• Failure to update asset records

======================================================================

REFERENCED LIBRARIES

DD-058 Assembly Standards Library

QC Library

ROOM Library

LAB Library

Asset Register

Customer Service Agreements

Digital Project Record

Manufacturer Maintenance Guidelines

======================================================================

REVISION APPROVAL

Service Manager Approval

______________________

Operations Approval

______________________

Effective Date

______________________

Supersedes Version

N/A

Change Summary

Initial Release

======================================================================

END OF STANDARD

DD-0601

======================================================================