======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-003

ROOM NAME:

Standard Bedroom

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Standard Bedroom serves as a secondary sleeping space while

providing the homeowner with the infrastructure necessary for

television, networking, and future technology expansion.

This standard establishes the required infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every secondary bedroom installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Standard Bedroom shall follow this standard.

======================================================================

ROOM FUNCTION

Secondary Bedroom

Optional Entertainment Area

Network Access Location

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1 Optional

(Installed when shown on approved floor plan.)

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

1

Home Run

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Standard Coverage

--------------------------------------------------

Dedicated Access Point

None

Unless required by engineered wireless coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Window Contact

--------------------------------------------------

Optional

Motion Sensor

If included in security package.

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-002

Single Cat6

--------------------------------------------------

Optional

ASM-TV-002

Television Connect Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Cat6 Installed

□ RG6 Installed (if applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Television Location

□ Additional Cat6

□ Additional RG6

□ Pair of Ceiling Speakers

□ TV Mount Installation

□ Concealed Wire Package

□ Gaming Network Package

□ Smart TV Connection Package

======================================================================

ROOM DESIGN RULES

1.

Every Standard Bedroom shall receive one ENT conduit.

--------------------------------------------------

2.

Every Standard Bedroom shall receive one Cat6 home

run as the company standard.

--------------------------------------------------

3.

RG6 shall only be installed when a television

location is included.

--------------------------------------------------

4.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

--------------------------------------------------

5.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-003

======================================================================