======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-004

ROOM NAME:

Home Office / Study

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Home Office serves as the primary work and productivity space

within the home.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Home Office installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Home Office shall follow this standard.

======================================================================

ROOM FUNCTION

Primary Work Area

Primary Data Location

Video Conference Ready

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

(Standard)

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed only when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Window Contact

--------------------------------------------------

Optional

Motion Sensor

If included in security package.

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Television Connect Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed (if applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional Cat6

□ Additional RG6

□ Television Location

□ Pair of Ceiling Speakers

□ Wireless Access Point

□ Network Switch

□ Fiber Conduit

□ Video Conference Package

□ Smart Office Package

======================================================================

ROOM DESIGN RULES

1.

Every Home Office shall receive one ENT conduit.

--------------------------------------------------

2.

Every Home Office shall receive two Cat6 home runs

as the company standard.

--------------------------------------------------

3.

RG6 shall only be installed when a television

location is included.

--------------------------------------------------

4.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

--------------------------------------------------

5.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-004

======================================================================