======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-005

ROOM NAME:

Media Room / Home Theater

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Media Room serves as the dedicated entertainment space within

the home.

This standard establishes the required infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Media Room or Home Theater installed by Digitally

Defined.

Unless specifically overridden by an approved floor plan revision,

every Media Room shall follow this standard.

======================================================================

ROOM FUNCTION

Dedicated Entertainment Room

Premium Audio Zone

Home Theater

Gaming & Streaming Space

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

(Primary Display)

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

4

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Equipment Conduit

1

2" ENT

From Display Location to Equipment Cabinet

(Optional based on room layout)

--------------------------------------------------

Equipment Location

Required

Cabinet or Equipment Rack

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

5.1 Speaker Prewire

--------------------------------------------------

Upgrade Option

7.1 Speaker Prewire

--------------------------------------------------

Upgrade Option

5.1.2 Dolby Atmos

--------------------------------------------------

Upgrade Option

5.1.4 Dolby Atmos

--------------------------------------------------

Upgrade Option

7.1.4 Dolby Atmos

--------------------------------------------------

Upgrade Option

Dual Subwoofer Prewire

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Sensor

======================================================================

ASSEMBLIES

Required

ASM-TV-002

Premium Television Location

--------------------------------------------------

Required

ASM-DATA-003

Quad Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-AUD-005

5.1 Theater Prewire

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-006

7.1 Expansion

--------------------------------------------------

Optional

ASM-AUD-007

Dolby Atmos Expansion

--------------------------------------------------

Optional

ASM-RACK-001

Equipment Rack

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

Required

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Equipment Conduit Installed

□ Four Cat6 Installed

□ RG6 Installed

□ Speaker Wire Installed

□ Speaker Locations Verified

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Equipment Cabinet Ready

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ 7.1 Speaker Package

□ Dolby Atmos Package

□ Additional Subwoofer

□ Equipment Rack

□ AV Receiver Installation

□ Projector Package

□ Projection Screen

□ Acoustic Treatment

□ Gaming Package

□ Streaming Equipment

□ Universal Remote

□ Smart Home Integration

======================================================================

ROOM DESIGN RULES

1.

Every Media Room shall receive four Cat6 home runs

as the company standard.

--------------------------------------------------

2.

Every Media Room shall receive one RG6 home run.

--------------------------------------------------

3.

Every Media Room shall receive one television conduit.

--------------------------------------------------

4.

Every Media Room shall receive one equipment

location.

--------------------------------------------------

5.

Speaker wiring shall be home-run to the equipment

location.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-005

======================================================================