======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-006

ROOM NAME:

Outdoor Living / Covered Patio

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Outdoor Living area serves as the primary outdoor entertainment

space for the home.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Covered Patio or Outdoor Living area installed by

Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Outdoor Living area shall follow this standard.

======================================================================

ROOM FUNCTION

Outdoor Entertainment

Outdoor Television

Outdoor Audio

Outdoor Wireless Coverage

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1 Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Weather-Rated

--------------------------------------------------

Equipment Location

None

(Default equipment located inside the home.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Outdoor Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

One Pair Outdoor Speakers

--------------------------------------------------

Upgrade Option

Two Pair Outdoor Speakers

--------------------------------------------------

Upgrade Option

Outdoor Subwoofer

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

Optional

Exterior Camera

Located per Exterior Camera Standard.

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Lighting Integration

--------------------------------------------------

Optional

Exterior Motion Detection

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Outdoor Television Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Optional

ASM-AUD-010

Outdoor Speaker Pair

--------------------------------------------------

Optional

ASM-AP-002

Outdoor Wireless Access Point

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed

□ Weather Protection Installed

□ Exterior Penetrations Sealed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Outdoor Television

□ Additional Cat6

□ Additional RG6

□ Outdoor Speaker Package

□ Outdoor Subwoofer

□ Outdoor Wireless Access Point

□ Exterior Camera

□ Landscape Audio

□ Outdoor Equipment Cabinet

□ Outdoor Smart Lighting Integration

======================================================================

ROOM DESIGN RULES

1.

Every Outdoor Living area shall receive one ENT

conduit.

--------------------------------------------------

2.

Every Outdoor Living area shall receive two Cat6

home runs as the company standard.

--------------------------------------------------

3.

Every Outdoor Living area shall receive one RG6

home run when a television location is included.

--------------------------------------------------

4.

All exterior penetrations shall be properly sealed

against moisture intrusion.

--------------------------------------------------

5.

Only weather-rated products shall be installed in

exterior environments.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-006

======================================================================