======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-007

ROOM NAME:

Bonus Room / Flex Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Bonus Room / Flex Room serves as a multi-purpose living space

that can function as a game room, playroom, secondary family room,

fitness room, hobby room, or future entertainment area.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Bonus Room or Flex Room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Bonus Room shall follow this standard.

======================================================================

ROOM FUNCTION

Multi-Purpose Living Area

Secondary Entertainment Space

Future Expansion Ready

Optional Home Theater

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed only when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Two Pair Ceiling Speakers

--------------------------------------------------

Upgrade Option

5.1 Surround Prewire

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Sensor

If included in security package.

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-AUD-005

5.1 Surround Prewire

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional TV Outlet

□ Additional Cat6

□ Additional RG6

□ Pair of Ceiling Speakers

□ Two Pair Ceiling Speakers

□ Home Theater Package

□ Gaming Package

□ TV Mount Installation

□ Concealed Wire Package

□ Smart Room Integration

======================================================================

ROOM DESIGN RULES

1.

Every Bonus Room shall contain one standard

television location.

--------------------------------------------------

2.

Every Bonus Room shall receive two Cat6 home runs

as the company standard.

--------------------------------------------------

3.

Every Bonus Room shall receive one RG6 home run as

the company standard.

--------------------------------------------------

4.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

--------------------------------------------------

5.

Equipment shall not be located inside the room unless

specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-007

======================================================================