======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-008

ROOM NAME:

Loft / Upstairs Living Area

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Loft / Upstairs Living Area serves as a secondary family

gathering space that supports television, gaming, streaming,

studying, and general entertainment.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Loft or Upstairs Living Area installed by Digitally

Defined.

Unless specifically overridden by an approved floor plan revision,

every Loft shall follow this standard.

======================================================================

ROOM FUNCTION

Secondary Living Room

Family Entertainment Area

Gaming Area

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Two Pair Ceiling Speakers

--------------------------------------------------

Upgrade Option

5.1 Surround Prewire

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Sensor

If included in security package.

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-AUD-005

5.1 Surround Prewire

--------------------------------------------------

Optional

ASM-AP-001

Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional TV Outlet

□ Additional Cat6

□ Additional RG6

□ Pair of Ceiling Speakers

□ Home Theater Package

□ Gaming Network Package

□ Wireless Access Point

□ TV Mount Installation

□ Concealed Wire Package

□ Smart Room Integration

======================================================================

ROOM DESIGN RULES

1.

Every Loft shall contain one standard television

location.

--------------------------------------------------

2.

Every Loft shall receive two Cat6 home runs as the

company standard.

--------------------------------------------------

3.

Every Loft shall receive one RG6 home run as the

company standard.

--------------------------------------------------

4.

Wireless coverage shall be evaluated for every

second-story Loft.

--------------------------------------------------

5.

Equipment shall not be located inside the room unless

specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-008

======================================================================