======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-009

ROOM NAME:

Kitchen

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Kitchen serves as the central gathering and food preparation

area within the home while supporting modern connectivity,

entertainment, and smart home technology.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Kitchen installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Kitchen shall follow this standard.

======================================================================

ROOM FUNCTION

Food Preparation Area

Family Gathering Space

Smart Home Control Location

Network Connectivity Area

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

--------------------------------------------------

ENT Conduit

None

Unless television location is specified.

--------------------------------------------------

Cat6

1

Home Run

(Location determined by approved floor plan.)

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only with television package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed only when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Panic Button

(Optional by customer request.)

======================================================================

SMART HOME OPTIONS

Optional

Smart Display Location

--------------------------------------------------

Optional

Voice Assistant Location

--------------------------------------------------

Optional

Smart Appliance Network Connection

======================================================================

ASSEMBLIES

Required

ASM-DATA-002

Single Cat6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-TV-002

Kitchen Television

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-AP-001

Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Cat6 Installed

□ Television Wiring Installed (If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Cable Protection

□ Smart Device Location Verified

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Kitchen Television

□ Additional Cat6

□ Wireless Access Point

□ Ceiling Speaker Package

□ Smart Display

□ Voice Assistant

□ Smart Appliance Connections

□ Whole Home Audio Integration

======================================================================

ROOM DESIGN RULES

1.

Every Kitchen shall receive one Cat6 home run as

the company standard.

--------------------------------------------------

2.

Television wiring shall only be installed when shown

on the approved floor plan.

--------------------------------------------------

3.

Wireless coverage shall always be verified for the

Kitchen area.

--------------------------------------------------

4.

Equipment shall not be installed inside the Kitchen

unless specified by the package.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

Kitchen technology shall remain unobtrusive and

maintain the finished appearance of the home.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-009

======================================================================