======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-010

ROOM NAME:

Dining Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Dining Room serves as a formal or casual dining space while

supporting future technology expansion, distributed audio, and

wireless network coverage.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Dining Room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Dining Room shall follow this standard.

======================================================================

ROOM FUNCTION

Dining Area

Conversation Space

Distributed Audio Zone

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

None

(Not included as standard.)

--------------------------------------------------

ENT Conduit

None

--------------------------------------------------

Cat6

None

(Standard installation does not require data.)

--------------------------------------------------

RG6

None

--------------------------------------------------

Power Bridge

None

--------------------------------------------------

Equipment Location

None

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Standard Coverage

--------------------------------------------------

Dedicated Access Point

None

Unless required by engineered wireless coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

--------------------------------------------------

Upgrade Option

Multi-Zone Audio

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

None

======================================================================

SMART HOME OPTIONS

Optional

Voice Assistant Location

--------------------------------------------------

Optional

Smart Lighting Control

======================================================================

ASSEMBLIES

Required

ASM-LABEL-001

Room Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-AUD-003

Multi-Room Audio

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Speaker Wire Installed (If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Ceiling Speaker Locations Verified

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Pair of Ceiling Speakers

□ Whole Home Audio

□ Smart Lighting Integration

□ Voice Assistant

□ Dining Room Television

□ Decorative Audio Controls

======================================================================

ROOM DESIGN RULES

1.

Dining Rooms shall remain visually clean with minimal

visible technology.

--------------------------------------------------

2.

Technology shall support entertainment without

impacting room aesthetics.

--------------------------------------------------

3.

Distributed audio shall use approved speaker

locations.

--------------------------------------------------

4.

Only approved assemblies may be installed.

--------------------------------------------------

5.

Wireless coverage shall be maintained throughout the

Dining Room.

--------------------------------------------------

6.

All optional technology shall integrate with the

approved package selected for the home.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-010

======================================================================