======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-013

ROOM NAME:

Primary Bathroom

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Primary Bathroom serves as the homeowner's primary personal

care space while supporting modern smart home technology and future

expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Primary Bathroom installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Primary Bathroom shall follow this standard.

======================================================================

ROOM FUNCTION

Primary Bathroom

Smart Home Support

Audio Zone

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

(Mirror TV or Wall TV)

--------------------------------------------------

ENT Conduit

Optional

1" ENT

Installed only when television is specified.

--------------------------------------------------

Cat6

1

Home Run

(Standard Smart Device Location)

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when television package is selected.

--------------------------------------------------

Equipment Location

None

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

None

Unless required by engineered wireless coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Moisture-Rated Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

Interior cameras are prohibited in bathrooms.

======================================================================

SECURITY REQUIREMENTS

Optional

Emergency Panic Button

--------------------------------------------------

Optional

Water Leak Detection

======================================================================

SMART HOME OPTIONS

Optional

Smart Mirror

--------------------------------------------------

Optional

Smart Exhaust Fan

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Heated Floor Control

======================================================================

ASSEMBLIES

Required

ASM-DATA-002

Single Cat6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-TV-003

Bathroom Television

--------------------------------------------------

Optional

ASM-AUD-011

Moisture-Rated Ceiling Speakers

--------------------------------------------------

Optional

ASM-SMART-002

Smart Bathroom Devices

--------------------------------------------------

Optional

ASM-SEC-005

Water Leak Detection

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Cat6 Installed

□ ENT Installed (If Applicable)

□ RG6 Installed (If Applicable)

□ Speaker Wiring Installed

(If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Bathroom Television

□ Smart Mirror

□ Moisture-Rated Ceiling Speakers

□ Voice Assistant

□ Heated Floor Controller

□ Water Leak Detection

□ Additional Cat6

□ Smart Exhaust Fan

======================================================================

ROOM DESIGN RULES

1.

Every Primary Bathroom shall receive one Cat6 home

run as the company standard.

--------------------------------------------------

2.

Interior surveillance devices are prohibited.

--------------------------------------------------

3.

Only moisture-rated products shall be installed

within wet environments.

--------------------------------------------------

4.

Technology shall remain discreet and complement the

finished appearance of the room.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-013

======================================================================