======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-019

ROOM NAME:

Home Gym / Fitness Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Home Gym serves as a dedicated fitness and wellness space while

providing the technology infrastructure required for streaming,

connected fitness equipment, audio, and future expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Home Gym installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Home Gym shall follow this standard.

======================================================================

ROOM FUNCTION

Fitness Area

Streaming Media

Connected Exercise Equipment

Wellness Space

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Dual-Zone Audio

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

Interior cameras installed only by homeowner request.

======================================================================

SECURITY REQUIREMENTS

None

======================================================================

SMART HOME OPTIONS

Optional

Smart Fitness Display

--------------------------------------------------

Optional

Connected Equipment Network

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Environmental Monitoring

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-AP-001

Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Television Installation

□ Additional Cat6

□ Additional RG6

□ Ceiling Speaker Package

□ Wireless Access Point

□ Connected Fitness Equipment

□ Voice Assistant

□ Smart Mirror

□ Environmental Sensors

======================================================================

ROOM DESIGN RULES

1.

Every Home Gym shall receive one television location.

--------------------------------------------------

2.

Every Home Gym shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Every Home Gym shall receive one RG6 home run as the

company standard.

--------------------------------------------------

4.

Wireless coverage shall support streaming fitness

equipment throughout the room.

--------------------------------------------------

5.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-019

======================================================================