======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-020

ROOM NAME:

Game Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Game Room serves as a dedicated entertainment area supporting

gaming systems, streaming media, television, distributed audio,

and future technology expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Game Room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Game Room shall follow this standard.

======================================================================

ROOM FUNCTION

Entertainment Area

Gaming Center

Secondary Living Space

Multi-Purpose Recreation Room

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

(Standard)

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

4

Home Run

(Two at TV Location,

Two at Gaming Location)

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

Optional

Gaming Cabinet or AV Cabinet

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

5.1 Surround Sound

--------------------------------------------------

Upgrade Option

7.1 Surround Sound

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

None

======================================================================

SMART HOME OPTIONS

Optional

Gaming Network Optimization

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Smart Lighting Scenes

--------------------------------------------------

Optional

Motorized Shades

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-003

Quad Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-005

5.1 Surround Prewire

--------------------------------------------------

Optional

ASM-AUD-006

7.1 Surround Prewire

--------------------------------------------------

Optional

ASM-AP-001

Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Four Cat6 Installed

□ One RG6 Installed

□ Speaker Wiring Installed

(If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Gaming Network Package

□ Additional Cat6

□ Additional RG6

□ Ceiling Speaker Package

□ 5.1 Surround Sound

□ 7.1 Surround Sound

□ Wireless Access Point

□ Smart Lighting Scenes

□ Voice Assistant

□ Motorized Shades

======================================================================

ROOM DESIGN RULES

1.

Every Game Room shall receive four Cat6 home runs

as the company standard.

--------------------------------------------------

2.

Every Game Room shall receive one television

location.

--------------------------------------------------

3.

Every Game Room shall receive one RG6 home run.

--------------------------------------------------

4.

Wireless coverage shall support multiple gaming

devices simultaneously.

--------------------------------------------------

5.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-020

======================================================================