======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-021

ROOM NAME:

Nursery

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Nursery serves as an infant or young child's bedroom while

providing safe, expandable technology infrastructure that supports

future growth without requiring additional construction.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Nursery installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Nursery shall follow this standard.

======================================================================

ROOM FUNCTION

Infant Bedroom

Child Bedroom

Future Bedroom

Smart Monitoring Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

Optional

Nursery Monitoring Camera Prewire

(Customer supplied monitoring device.)

======================================================================

SECURITY REQUIREMENTS

Optional

Window Contact

--------------------------------------------------

Optional

Glass Break Sensor

======================================================================

SMART HOME OPTIONS

Optional

Baby Monitoring Station

--------------------------------------------------

Optional

Smart Lighting Scenes

--------------------------------------------------

Optional

Motorized Shades

--------------------------------------------------

Optional

Environmental Sensor

(Temperature / Humidity)

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Television Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Optional

ASM-CAM-003

Nursery Camera Prewire

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed (If Applicable)

□ Camera Prewire Installed

(If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Television Location

□ Additional Cat6

□ Additional RG6

□ Nursery Camera Prewire

□ Ceiling Speaker Package

□ Smart Lighting Scenes

□ Motorized Shades

□ Environmental Monitoring

□ Voice Assistant

======================================================================

ROOM DESIGN RULES

1.

Every Nursery shall receive one ENT conduit for

future expansion.

--------------------------------------------------

2.

Every Nursery shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Technology shall support future room conversion into

a standard bedroom without rewiring.

--------------------------------------------------

4.

Camera prewiring shall terminate at the structured

wiring location.

--------------------------------------------------

5.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-021

======================================================================