======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-022

ROOM NAME:

Guest Suite

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Guest Suite serves as a private accommodation for visitors or

multi-generational living while providing technology

infrastructure equivalent to a premium bedroom.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Guest Suite installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Guest Suite shall follow this standard.

======================================================================

ROOM FUNCTION

Guest Bedroom

Secondary Living Space

Entertainment Area

Future Primary Bedroom

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Window Contact

--------------------------------------------------

Optional

Glass Break Sensor

--------------------------------------------------

Optional

Motion Sensor

======================================================================

SMART HOME OPTIONS

Optional

Voice Assistant

--------------------------------------------------

Optional

Motorized Shades

--------------------------------------------------

Optional

Smart Lighting Scenes

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional TV Outlet

□ Additional Cat6

□ Additional RG6

□ Ceiling Speaker Package

□ Voice Assistant

□ Motorized Shades

□ Smart Lighting Scenes

□ TV Mount Installation

□ Concealed Wire Package

======================================================================

ROOM DESIGN RULES

1.

Every Guest Suite shall contain one standard

television location.

--------------------------------------------------

2.

Every Guest Suite shall receive two Cat6 home runs

as the company standard.

--------------------------------------------------

3.

Every Guest Suite shall receive one RG6 home run as

the company standard.

--------------------------------------------------

4.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

--------------------------------------------------

5.

Equipment shall not be located inside the room unless

specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-022

======================================================================