======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-023

ROOM NAME:

Casita / In-Law Suite

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Casita / In-Law Suite serves as a self-contained living area

that supports independent occupancy while maintaining full

integration with the home's technology infrastructure.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Casita or In-Law Suite installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Casita shall follow this standard.

======================================================================

ROOM FUNCTION

Independent Living Area

Guest Residence

Multi-Generational Living

Private Entertainment Space

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

2

(Living Area and Bedroom)

--------------------------------------------------

ENT Conduit

2

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

4

Home Run

--------------------------------------------------

RG6

2

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package.

--------------------------------------------------

Equipment Location

Optional

Mini Structured Wiring Cabinet

(If specified by approved floor plan.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Living Area Ceiling Speakers

--------------------------------------------------

Upgrade Option

Bedroom Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

Exterior cameras reference Exterior Camera Standards.

======================================================================

SECURITY REQUIREMENTS

Standard

Door Contact

--------------------------------------------------

Optional

Motion Sensor

--------------------------------------------------

Optional

Glass Break Sensor

======================================================================

SMART HOME OPTIONS

Optional

Voice Assistant

--------------------------------------------------

Optional

Smart Thermostat

--------------------------------------------------

Optional

Motorized Shades

--------------------------------------------------

Optional

Smart Lighting Scenes

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

(Qty 2)

--------------------------------------------------

Required

ASM-DATA-003

Quad Cat6

--------------------------------------------------

Required

ASM-RG6-002

Dual RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Ceiling Speakers

--------------------------------------------------

Optional

ASM-AP-001

Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Two ENT Installed

□ Four Cat6 Installed

□ Two RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Heights

□ Equipment Location Verified

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional Television Location

□ Additional Cat6

□ Additional RG6

□ Ceiling Speaker Package

□ Wireless Access Point

□ Smart Thermostat

□ Voice Assistant

□ Motorized Shades

□ Smart Lighting Control

□ Mini Equipment Cabinet

======================================================================

ROOM DESIGN RULES

1.

Every Casita shall function as a complete living

space with independent technology infrastructure.

--------------------------------------------------

2.

Every Casita shall receive four Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Every Casita shall receive two RG6 home runs.

--------------------------------------------------

4.

Wireless coverage shall fully support independent

occupancy.

--------------------------------------------------

5.

Equipment shall remain integrated with the home's

primary technology system unless otherwise approved.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-023

======================================================================