======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-025

ROOM NAME:

Home Bar / Wet Bar

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Home Bar / Wet Bar serves as an entertainment area supporting

television, distributed audio, smart lighting, and future

hospitality technologies.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Home Bar or Wet Bar installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Home Bar shall follow this standard.

======================================================================

ROOM FUNCTION

Entertainment Area

Hospitality Space

Audio Zone

Smart Home Integration

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1 Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

(If television location is included.)

--------------------------------------------------

Power Bridge

Optional

Installed only when television package is selected.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Dual Pair Ceiling Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

None

======================================================================

SMART HOME OPTIONS

Optional

Smart Lighting Scenes

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Touchscreen Controller

--------------------------------------------------

Optional

Motorized Accent Lighting

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Television Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed (If Applicable)

□ Speaker Wiring Installed

(If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Television Location

□ Additional Cat6

□ Additional RG6

□ Pair of Ceiling Speakers

□ Dual Pair Ceiling Speakers

□ Smart Lighting Control

□ Voice Assistant

□ Touchscreen Controller

□ Motorized Accent Lighting

======================================================================

ROOM DESIGN RULES

1.

Every Home Bar shall receive one ENT conduit for

future expansion.

--------------------------------------------------

2.

Every Home Bar shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Every Home Bar shall receive one RG6 home run when

a television location is included.

--------------------------------------------------

4.

Technology shall remain concealed whenever practical.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-025

======================================================================