======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-026

ROOM NAME:

Library / Reading Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Library / Reading Room serves as a quiet space for reading,

studying, and personal productivity while providing discreet

technology infrastructure for networking, audio, and future smart

home expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Library or Reading Room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Library shall follow this standard.

======================================================================

ROOM FUNCTION

Reading Area

Study Space

Quiet Workspace

Future Flex Room

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

Optional

Installed only when television location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when television package is selected.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Bookshelf Speaker Prewire

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

None

======================================================================

SECURITY REQUIREMENTS

Optional

Glass Break Sensor

======================================================================

SMART HOME OPTIONS

Optional

Voice Assistant

--------------------------------------------------

Optional

Motorized Shades

--------------------------------------------------

Optional

Scene Lighting Control

--------------------------------------------------

Optional

Smart Reading Display

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Television Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Optional

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed (If Applicable)

□ Speaker Wiring Installed

(If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Television Location

□ Additional Cat6

□ Additional RG6

□ Pair of Ceiling Speakers

□ Bookshelf Speaker Prewire

□ Voice Assistant

□ Motorized Shades

□ Scene Lighting Control

□ Smart Reading Display

======================================================================

ROOM DESIGN RULES

1.

Every Library shall receive one ENT conduit for

future expansion.

--------------------------------------------------

2.

Every Library shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Technology shall remain discreet to preserve the

quiet character of the room.

--------------------------------------------------

4.

Equipment shall not be installed inside the room

unless specified by the package.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-026

======================================================================