======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-028

ROOM NAME:

Theater Lobby / Entertainment Hall

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Theater Lobby / Entertainment Hall serves as the transition

space between the home's entertainment areas while supporting

distributed audio, digital signage, smart lighting, and future

technology expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Theater Lobby or Entertainment Hall installed by

Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Theater Lobby shall follow this standard.

======================================================================

ROOM FUNCTION

Entertainment Transition Area

Gathering Space

Audio Zone

Smart Lighting Zone

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Optional

(Digital Signage or Display)

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

Optional

Installed only when display location is included.

--------------------------------------------------

Power Bridge

Optional

Installed only when display package is selected.

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured wiring panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

Distributed Audio Zone

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

======================================================================

CAMERA REQUIREMENTS

Optional

Interior Camera

(Customer approval required.)

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Sensor

======================================================================

SMART HOME OPTIONS

Optional

Scene Lighting Controller

--------------------------------------------------

Optional

Touchscreen Controller

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Digital Welcome Display

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-AUD-001

Pair of Ceiling Speakers

--------------------------------------------------

Optional

ASM-TV-002

Display Location

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ Speaker Wiring Installed

□ RG6 Installed (If Applicable)

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Digital Display

□ Additional Cat6

□ Additional RG6

□ Distributed Audio

□ Smart Lighting Scenes

□ Touchscreen Controller

□ Voice Assistant

□ Interior Camera

======================================================================

ROOM DESIGN RULES

1.

Every Theater Lobby shall receive one ENT conduit

for future expansion.

--------------------------------------------------

2.

Every Theater Lobby shall receive two Cat6 home runs

as the company standard.

--------------------------------------------------

3.

Every Theater Lobby shall receive one pair of ceiling

speakers as the default entertainment standard.

--------------------------------------------------

4.

Technology shall complement the luxury appearance of

the entertainment area.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-028

======================================================================