======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-035

ROOM NAME:

Balcony / Upper Deck

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Balcony / Upper Deck serves as an elevated outdoor living space

that supports entertainment, wireless connectivity, security

surveillance, and future outdoor technology expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Balcony or Upper Deck installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Balcony shall follow this standard.

======================================================================

ROOM FUNCTION

Outdoor Living Space

Entertainment Area

Observation Area

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1 Optional

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

(If television location is included.)

--------------------------------------------------

Power Bridge

Optional

Weather-Rated

--------------------------------------------------

Equipment Location

None

(Default equipment located inside the home.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Outdoor Access Point

Recommended

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Outdoor Speakers

--------------------------------------------------

Upgrade Option

Whole Home Audio Integration

--------------------------------------------------

Upgrade Option

Outdoor Subwoofer

======================================================================

CAMERA REQUIREMENTS

Optional

Balcony Security Camera

--------------------------------------------------

Optional

Perimeter Observation Camera

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Detection

--------------------------------------------------

Optional

Perimeter Monitoring

======================================================================

SMART HOME OPTIONS

Optional

Outdoor Lighting Control

--------------------------------------------------

Optional

Motorized Privacy Screens

--------------------------------------------------

Optional

Voice Assistant

--------------------------------------------------

Optional

Outdoor Climate Sensors

======================================================================

ASSEMBLIES

Required

ASM-TUBE-001

Standard ENT Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Optional

ASM-TV-002

Outdoor Television

--------------------------------------------------

Optional

ASM-RG6-001

Single RG6

--------------------------------------------------

Optional

ASM-AUD-010

Outdoor Speaker Pair

--------------------------------------------------

Optional

ASM-AP-002

Outdoor Wireless Access Point

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ RG6 Installed (If Applicable)

□ Weather-Rated Devices Installed

□ Exterior Penetrations Sealed

□ Proper Labeling

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Outdoor Television

□ Outdoor Speaker Package

□ Outdoor Subwoofer

□ Wireless Access Point

□ Security Camera

□ Smart Lighting

□ Motorized Privacy Screens

□ Voice Assistant

□ Climate Sensors

======================================================================

ROOM DESIGN RULES

1.

Every Balcony shall receive one ENT conduit for

future expansion.

--------------------------------------------------

2.

Every Balcony shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Only weather-rated products shall be installed in

exterior environments.

--------------------------------------------------

4.

All exterior penetrations shall be sealed against

water intrusion.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-035

======================================================================