======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-036

ROOM NAME:

Pool Equipment Area

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Pool Equipment Area serves as the central location for pool

automation, equipment control, monitoring, and future expansion.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Pool Equipment Area installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Pool Equipment Area shall follow this standard.

======================================================================

ROOM FUNCTION

Pool Automation

Equipment Control

Utility Infrastructure

Future Expansion Ready

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

None

--------------------------------------------------

ENT Conduit

2

2" ENT

Future Expansion

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

None

--------------------------------------------------

Power Bridge

None

--------------------------------------------------

Equipment Location

Pool Automation Panel

(Customer or Builder Supplied)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Dedicated Outdoor Access Point

Optional

Installed when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

======================================================================

CAMERA REQUIREMENTS

Optional

Pool Equipment Camera

======================================================================

SECURITY REQUIREMENTS

Optional

Equipment Gate Contact

--------------------------------------------------

Optional

Tamper Sensor

======================================================================

SMART HOME OPTIONS

Optional

Pool Automation Controller

--------------------------------------------------

Optional

Smart Pump Monitoring

--------------------------------------------------

Optional

Chemical Monitoring

--------------------------------------------------

Optional

Leak Detection

--------------------------------------------------

Optional

Freeze Protection Monitoring

======================================================================

ASSEMBLIES

Required

ASM-TUBE-003

Future Expansion Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-SMART-009

Pool Automation

--------------------------------------------------

Optional

ASM-CAM-007

Equipment Area Camera

--------------------------------------------------

Optional

ASM-AP-002

Outdoor Wireless Access Point

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

If Applicable

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Expansion Conduits Installed

□ Two Cat6 Installed

□ Equipment Location Verified

□ Exterior Devices Weather Protected

□ Exterior Penetrations Sealed

□ Proper Labeling

□ Automation Wiring Installed

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Pool Automation System

□ Smart Pump Controller

□ Chemical Monitoring

□ Leak Detection

□ Equipment Camera

□ Wireless Access Point

□ Additional Expansion Conduit

□ Freeze Protection Monitoring

======================================================================

ROOM DESIGN RULES

1.

Every Pool Equipment Area shall receive two future

expansion conduits.

--------------------------------------------------

2.

Every Pool Equipment Area shall receive two Cat6

home runs as the company standard.

--------------------------------------------------

3.

Technology shall remain accessible for future

service and maintenance.

--------------------------------------------------

4.

All exterior devices shall be weather-rated.

--------------------------------------------------

5.

Only approved assemblies may be installed.

--------------------------------------------------

6.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-036

======================================================================