======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-040

ROOM NAME:

Future Expansion / Unfinished Space

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Future Expansion / Unfinished Space serves as a reserved area

within the home intended for future build-out without requiring

major structural modifications or rewiring.

This standard establishes the required technology infrastructure,

installation requirements, assembly assignments, and expansion paths

for every unfinished room, attic bonus space, basement finish area,

or designated future room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Future Expansion area shall follow this standard.

======================================================================

ROOM FUNCTION

Future Living Space

Future Bedroom

Future Office

Future Theater

Future Flex Room

Technology Expansion Zone

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

Future Only

--------------------------------------------------

ENT Conduit

Minimum (2)

2" ENT

Home Run to Structured Wiring Location

--------------------------------------------------

Cat6

2

Future Home Runs

Coiled and Labeled

--------------------------------------------------

RG6

1

Future Home Run

Coiled and Labeled

--------------------------------------------------

Speaker Wire

Optional

Future Audio

--------------------------------------------------

Equipment Location

None

Reserved for Future Design

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Planned

--------------------------------------------------

Dedicated Access Point

Reserved Location

Only installed when room is completed.

======================================================================

AUDIO OPTIONS

Reserved

Pair of Ceiling Speakers

--------------------------------------------------

Reserved

Home Theater

--------------------------------------------------

Reserved

Whole Home Audio

======================================================================

CAMERA REQUIREMENTS

Reserved

As Required

======================================================================

SECURITY REQUIREMENTS

Reserved

Motion Sensor

--------------------------------------------------

Reserved

Window Contacts

======================================================================

SMART HOME OPTIONS

Reserved

Voice Assistant

--------------------------------------------------

Reserved

Lighting Control

--------------------------------------------------

Reserved

Motorized Shades

--------------------------------------------------

Reserved

Environmental Sensors

======================================================================

ASSEMBLIES

Required

ASM-TUBE-003

Future Expansion Conduit

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Future Speaker Prewire

======================================================================

LABOR

Automatically Generated

Prewire

Only

--------------------------------------------------

Trim

Future

--------------------------------------------------

Equipment

Future

--------------------------------------------------

Programming

Future

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ Expansion Conduits Installed

□ Pull Strings Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Future Wiring Coiled

□ Cable Labels Installed

□ Documentation Updated

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Room Conversion Package

□ Additional Expansion Conduit

□ Fiber Backbone

□ Additional Cat6

□ Additional RG6

□ Speaker Prewire

□ Access Point

□ Smart Home Expansion

□ Security Expansion

======================================================================

ROOM DESIGN RULES

1.

Every Future Expansion area shall receive a minimum

of two expansion conduits.

--------------------------------------------------

2.

Every Future Expansion area shall receive two Cat6

home runs.

--------------------------------------------------

3.

Every Future Expansion area shall receive one RG6

home run.

--------------------------------------------------

4.

All future wiring shall be properly labeled and

documented.

--------------------------------------------------

5.

Future infrastructure shall remain accessible after

construction.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

ROOM STANDARD LIBRARY COMPLETE

ROOM-001  Great Room

ROOM-002  Primary Bedroom

ROOM-003  Standard Bedroom

ROOM-004  Home Office

ROOM-005  Media Room

ROOM-006  Outdoor Living

ROOM-007  Bonus Room

ROOM-008  Loft

ROOM-009  Kitchen

ROOM-010  Dining Room

ROOM-011  Garage

ROOM-012  Laundry

ROOM-013  Primary Bathroom

ROOM-014  Standard Bathroom

ROOM-015  Foyer

ROOM-016  Pantry

ROOM-017  Mud Room

ROOM-018  Mechanical Room

ROOM-019  Home Gym

ROOM-020  Game Room

ROOM-021  Nursery

ROOM-022  Guest Suite

ROOM-023  Casita

ROOM-024  Outdoor Kitchen

ROOM-025  Home Bar

ROOM-026  Library

ROOM-027  Wine Room

ROOM-028  Theater Lobby

ROOM-029  Hallway

ROOM-030  Stairway

ROOM-031  Front Porch

ROOM-032  Rear Yard

ROOM-033  Front Yard

ROOM-034  Driveway

ROOM-035  Balcony

ROOM-036  Pool Equipment

ROOM-037  Gate / Entry

ROOM-038  Roof / Attic

ROOM-039  Structured Wiring Closet

ROOM-040  Future Expansion

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

DD-063 Installation Production Manual

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-040

======================================================================