======================================================================

DIGITALLY DEFINED

ROOM STANDARDS LIBRARY

ROOM STANDARD

ROOM ID:

ROOM-001

ROOM NAME:

Great Room

VERSION:

1.0

STATUS:

FROZEN

======================================================================

PURPOSE

The Great Room serves as the primary entertainment space within

the home.

This standard establishes the default technology infrastructure,

installation requirements, assembly assignments, and upgrade paths

for every Great Room installed by Digitally Defined.

Unless specifically overridden by an approved floor plan revision,

every Great Room shall follow this standard.

======================================================================

ROOM FUNCTION

Primary Entertainment Area

Primary Television Location

Primary Network Usage

Primary Family Gathering Space

======================================================================

STANDARD INFRASTRUCTURE

Television Locations

1

--------------------------------------------------

ENT Conduit

1

2" ENT

Home Run to Structured Wiring Panel

--------------------------------------------------

Cat6

2

Home Run

--------------------------------------------------

RG6

1

Home Run

--------------------------------------------------

Power Bridge

Optional

Installed only when included in package

--------------------------------------------------

Equipment Location

None

(Default equipment located in structured panel.)

======================================================================

NETWORK REQUIREMENTS

Wireless Coverage

Required

--------------------------------------------------

Access Point

Installed only when required by engineered wireless

coverage plan.

======================================================================

AUDIO OPTIONS

Builder Standard

None

--------------------------------------------------

Upgrade Option

Pair of Ceiling Speakers

--------------------------------------------------

Upgrade Option

5.1 Surround Prewire

--------------------------------------------------

Upgrade Option

7.1 Surround Prewire

--------------------------------------------------

Upgrade Option

Dolby Atmos Prewire

======================================================================

CAMERA REQUIREMENTS

None

Exterior cameras reference exterior room standards.

======================================================================

SECURITY REQUIREMENTS

Optional

Motion Sensor

If required by builder package.

======================================================================

ASSEMBLIES

Required

ASM-TV-001

Standard TV Infrastructure

--------------------------------------------------

Required

ASM-DATA-001

Dual Cat6

--------------------------------------------------

Required

ASM-RG6-001

Single RG6

--------------------------------------------------

Required

ASM-LABEL-001

Cable Identification

--------------------------------------------------

Optional

ASM-AUD-001

Ceiling Speakers

--------------------------------------------------

Optional

ASM-AUD-005

Home Theater Prewire

======================================================================

LABOR

Automatically Generated

Prewire

From Assembly Library

--------------------------------------------------

Trim

From Assembly Library

--------------------------------------------------

Equipment

From Package Library

--------------------------------------------------

Programming

If Applicable

======================================================================

PRODUCTS

Automatically Generated

No manual product selection permitted.

======================================================================

QUALITY CONTROL

Verify

□ ENT Installed

□ Pull String Installed

□ Two Cat6 Installed

□ One RG6 Installed

□ Proper Stapling

□ Proper Bend Radius

□ Proper Labeling

□ Wall Plate Height

□ Cable Protection

□ Photo Documentation

======================================================================

AVAILABLE UPGRADES

□ Additional TV Outlet

□ Additional Cat6

□ Additional RG6

□ Ceiling Speaker Package

□ Home Theater Package

□ Soundbar Power Bridge

□ TV Mount Installation

□ Fireplace TV Package

□ Concealed Wire Package

======================================================================

ROOM DESIGN RULES

1.

Every Great Room shall contain one primary television

location.

--------------------------------------------------

2.

Every Great Room shall receive two Cat6 home runs as

the company standard.

--------------------------------------------------

3.

Every Great Room shall receive one RG6 home run as

the company standard.

--------------------------------------------------

4.

All wiring shall terminate in the structured wiring

location unless otherwise engineered.

--------------------------------------------------

5.

Equipment shall not be located inside the room unless

specified by the package.

--------------------------------------------------

6.

Only approved assemblies may be installed.

======================================================================

RELATED DOCUMENTS

DD-055 Floor Plan Library

DD-056 Builder Package Catalog

DD-058 Assembly Standards Library

DD-059 Upgrade Catalog

======================================================================

REVISION HISTORY

Version 1.0

Initial Release

======================================================================

END OF ROOM STANDARD

ROOM-001

======================================================================